Response to Robert X. Cringley's “Notes from the Field” in 7/24/95 Infoworld (Volume 17, Issue 30)

Recently there have been several reports, both on the Internet and in the press, of an issue known as the “sniper bug.”  This happens on a network with both UNIX computers and a computer running a Microsoft protected mode TCP/IP protocol stack and where the Microsoft network computer is using the SMC Elite Ultra network interface card (NIC).  In this case, TCP/IP connections between the UNIX computers or between a Microsoft network computer and UNIX computer are lost.  This issue arises because the SMC

Elite Ultra NIC incorrectly passes up ethernet packets addressed to other computers. This causes the Microsoft TCP/IP protocol stack to send an ICMP Host Unreachable message back to the source

computer. Some UNIX TCP/IP stacks disconnect when they receive this ICMP packet.

To work around the SMC hardware problem, Microsoft has removed the code from the protected mode TCP/IP stack which sends the ICMP Host Unreachable message under these circumstances.  The following are the versions of TCP/IP that contain the fixed code:

- Windows 95: While this issue appeared in the beta of Windows 95, the workaround is included in the release version of Windows 95.

- Windows for Workgroups - ftp://ftp.microsoft.com/bussys/clients/wfw/tcpip32-fixes/icmpfix

Versions of the Windows for Workgroups TCP/IP-32 file, VIP.386, date stamped 7/19/95 or later                  incorporate the above workaround.

- Windows NT 3.5 - ftp://ftp.microsoft.com/bussys/winnt/winnt-unsup-ed/fixes/nt35/icmpfix

- Windows NT 3.51 - ftp://ftp.microsoft.com/bussys/winnt/winnt-unsup-ed/fixes/nt351/icmpfix

Versions of the Windows NT file, TCPIP.SYS, date stamped 6/14/95 or  later incorporate the above workaround.

The Elite Ultra is manufactured by SMC, a vendor independent of Microsoft; we make no warranty, implied or otherwise, regarding this product's performance or reliability.