Agenda

8:30	-9:30		Functional Overview & Architecture

9:30	-10:00		Installation

10:00-10:15		Break

10:15-11:30	Installation, Configuration and GUI’s

11:30 – 12:30	Lunch

12:30 - 			Actual installation

# The VST Team

Marketing

Kurt Merriweather		Product Manager

Louise Ledeen			Marketing Manager

Engineering

Ron Jacoby				Principal Engineer

Ashok Yerneni			Engineering Manager

Rick Reed				Technical Lead

Total 5 core engineers and 4 Q&A engineers

Applied Engineering

Mike Moskowitz			Manager Applied Engineering

Hilmi Ortadeveci			Applied Engineer

# VST Functional Overview & Architecture

- The following topics will be discussed in this module

## Functional Overview

What is VST and  where can it be used for

## Architecture

Overview of the design goals, architecure and the

VST modules

## Units

Units can be seen as virtuel VTR heads.

VST uses units to control Media Ports such as the DIVO.

# Functional Overview

- VST is designed to transform the Origin servers

into frame accurate and broadcast-quality video servers

- VST leverages the scaleabilty of the SGI Origin servers,  the video I/O boards and the digital media libraries

- We do the low-level IRIX and VL programming the developers embed VST in their end user applications

- It’s ment to give SGI developers a competetive advantage by enabling them to create very scaleable media and format indepent video server applications

- VST supports broadcast automations systems such as

Louth, making it an ideal platform  to be used in

broadcast environments

- VST can be used to build the following applications

NVOD

time delay

VTR replacement

add insertion

store and forward

new playout

# VST Workflow

## VST Workflow

Digital Media (Clips) are brought into the server by

recording

copying clips

retrieving it from archive

the VST-server van be controlled by

an application based upon MVCP (the VST-gui’s)

broadcast automation systems such as Louth, Odetics

Sony RS-422 protocol

to play out the clips to

broadcast system

a VTR

monitor

MPEG-2 decoder

stored on an archive system

# Design Goals

- Development platform

designed to be embedded in application

engine, not a UI

- Audio/Video record/playout platform

acquisition, distribution, play-to-air

not a full-featured editing tool

- Flexible, modular logic

no embedded application logic

plug-in modules

- Media independent

media-format independent

media-device independent

- Performance

real-time, frame-accurate control

configurable for low- and high-bandwidth

efficient data and control flow

- Native archive interoperability

# Software Overview

## Overview Software Components

- Core Software (vvtr)

provides the basic VST functionality

- Control Interface Modules

device-depending code to support control protocols (eg. Louth)

- Media Interfaces Modules

device-depending code to support media ports  (e.g., DIVO)

- Format Interface Modules

format-dependent code to support different media types

- Storage Interface Modules

provides access to storage systems

- Archive Interface Modules

provides native support to store, query and retrieve from archives

Core Software (vvtr)

- The core software provides the basic VST functionality for recording and playback

- It utilizes IRIX as well as portions of the SGI Digital Media Libraries

- The core software provides the following functionality

unit management, which links one or more external control

protocol modules (such as Louth) to the media ports

configuration management, which automatically configures the

VST software according to the hardware capabilities of the system

on which it runs

clip cache management, which maintains persistent information about the media that is either stored in the clip cache or is in the process of  being transferred into or out of it

archive management, which manages the transferring of assets into and out of an archive system

- Other provided utilities include

locking

logging

threading

- The core library supports external DSO’s (modules) which contain specific code for external entities

- When VST is started it loads and initializes all external modules which are installed

## Clip Cache

- Digital media data that VST processes for playout and recording is stored in one or more clip caches

- Each unit of data that is stored in a clip cache is called

a clip

- Clips can be added to the cache by

using VST to record the clip

recording clips on an other VST-server and adding them to the cache

transferring clips from a SC archive system

# Modules

Control Interface Module

- These modules translate to and from external control protocols

- The control interface modules allow various automation controllers and digital media applications to control VST

- The following control interface modules are provided

Louth VDCP

Sony RS-422

Odetics

MVCP

## Louth VDCP

- Louth Video Disk Communications Protocol defined by Louth Automation provides control of SVT over

RS-232

RS-422

TCP/IP

- The VST Louth interface module supports back-to-back playout, recording and archive management

Sony RS-422

- The Sony protocol is the standard for controlling VTRs

- The  Sony RS-422 VTR protocol is also called 9-pin or P2

- SVT supports this protocol through a VTR deck-emulation mode that includes frame-accurate insert editing and variable-speed shuttle

- The Sony protocol is not fully implemented

### Odetics

- The Odetics protocol is a Video Disk Control Protocol similar in purpose to the Louth VDCP

- Is built upon the foundation of the legacy of  Sony

RS-422

- Shares many commands and features with Sony RS-422

- Odetics adds to the Sony protocol commands specific to Video Disk Recorders, such as back-to-back playout, recording and clip management operations

### MVCP

- Multiple-Unit Video Computer Protocol (MVCP) is the native VST control protocol defined by SGI

- This control protocol provides full-featured control of VST through a TCP/IP connection

- This control interface module supports

archive management

multiple-unit control

event monitoring

provides access to advanced features of  SGI devices

Storage Device Interface Module

- Storage device interface modules provide access to the

storage systems on which the clip cache resides.

- Currently there is a storage device interface module for the IRIX XFS filesystem

Format Interface Module

- Format interface modules provide handlers for accessing specific digital media storage formats

- VST currently provides format interface modules that support the following

variable-frame format 	uncompressed, RICE and JPEG

stream-based format 		MPEG-2 transport and program streams

frame frame			DVCPRO Data Interchange Format

Media Device Interface Module

- Media device interface modules provide access to the media ports over which the media is played and recorded

- Each type of I/O port typically has its own media device interface module

- SVT has media device interface modules for the following

DIVO		support for DIVO and DIVO-DVC

O2video		support for O2 video

vela	  		support for the Vela Quad decoder

dvb-asi		support for the DVB-ASI card

diaquest		support for Sony RS-422

V-LAN 	support for V-LAN

Note: The Diaquest and V-LAN are media device

Interface modules which allow VST to control

videotape decks enabling frame-accurate

capture from, and lay-down to videotape decks

- When a developer wants to use a new media device the

SVT-engineering team needs to develop a new media device interface module

## Archive Interface Module

- These modules provide the support that is needed locate

a given clip in an archive system and bring it into VST and transfer clips into an archive

- SVT has an SC 2.0 archive interface module which enables VST to retrieve clips from, and store clips in, an archive asset repository

- VST transfers the media to and from an archive system

using the Asset Transfer Service (or ATS)

# Units

## Logical Playback and Record Units

- Logical units enable media ports to play and record clips

- Each VST unit can be thought of as a logical videotape recorder (VTR) transport that is capable of loading, cueing playing and recording clips using a specific media port

- Logical units are created automatically by VST if the VST GUI or an automation controller is used

- When the MVCP protocol is used an unit can be created with the UADD command or a unit (created by another control connection) can be opended with UOPEN

- There is normally a one-to-one relationship between a control connection to VST and a VST logical unit, and between a logical unit and a media port

- A single unit can also be controlled by multiple control

ports. For example, two tightly integrated applications

might control a single unit, where each application

would have its own control port.

- A media port can also be controlled by multiple logical units. For example, an application with one control connection and two units could be cueing one clip while playing out another, enabling back-to-back playout of clips when allowed by the media format

- A media port can have multiple (>2) units connected to it

- In case of a NETWORK media port  (e.g., DVB-ASI) each output channel will be a separate unit

## Installing VST

- The following topics will ne discussed in the section

Supported Hardware

Overview of the supported server, media and disk storage

hardware

Installing VST SW

Overview of the required SW for the varoius VST configurations. Also an overview of the VST subsystems

Setting up the Clip Cache

Creating the striped XLV is the most important part of  the installation. Both real-time and non-real time filesystems will be discussed and configuring GRIO

## Supported Hardware

- Origin 2000

- Origin 200

- Onyx2

- O2 workstation

NOTE: O2 is supported for development only!

- Target Platform is O200 with GIGAChannel

- Majority of the testing is done on O200

- Limited testing for O2000 and almost none

for Onyx2

Supported Media I/O Boards

Video Devices

- DIVO						XIO-based

- DIVO DVCPRO 			XIO-based

- Quad MPEG-2 Decoder 	SCSI-based

- DVB-ASI					PCI-based

- O2 Video	AV1 (Analog)		O2-AV-IO

- O2 Video	AV2 (Digital)		O2-AV-IO

Audio Devices

- IO6G						XIO-based

use the IO6G of a Onyx2 in a O2000

- RAD						PCI-based

AES		2 digital stereo pairs

ADAT	8 optical channels (24 bit)

Origin 200

### 1-4 Digital Video Option boards

### XT-DIVO or XT-DIVO-DVC

### 2P with GIGAChannel

- 1-8 Digital Video Option boards

### XT-DIVO or XT-DIVO-DVC

### 2P with GIGAChannel

- 1-3 Vela MPEG2 Quad decoders per SCSI bus

- 1-3 DVB boards

ViewGraphics MediaPump MP 403 DVB-ASI (Input/Output)

DVB-ASI playout only

###### Origin 2000 & Onyx2

### 1-8 Digital Video Option

### XT-DIVO or XT-DIVO-DVC

- 1-3 Vela MPEG2 4-port SCSI based decoders per bus

- Both Configs are not fully-qualified for production use

- Developer is responsible for system intergration testing

- The DVB-ASI card has never been tested on an

- Origin 2000

O2

- AV1 (Analog Video Option)

- AV2 (Digital Video Option)

- 1 Vela MPEG2 4-port SCSI decoder

- DEVELOPMENT ONLY!

- Not qualified for production

- Uncompressed currently not supported on O2

###### I/O SLOTS

- Origin 2000

| Node | XIO | PCI |
| --- | --- | --- |
| 1 | 4 | 2 ½ |
| 2 | 10 | 2 ½ |

Note: If the PCI shoebox isn’t used an additional

XIO slot will be available

- Origin 200

| Tower(s) | XBOX | XIO | PCI |
| --- | --- | --- | --- |
| 1 | 0 | 0 | 3 |
| 1 | 1 | 5 | 6 ½ |
| 2 | 1 | 5 | 9 ½ |
| 2 | 2 | 10 | 12 +2 x ½ |

- Note:	½ PCI slot means half-length PCI slot

- 1  PCI slot means full-length PCI slot

- The DVB-ASI board needs full-length PCI slot

Disk Storage Options

- For O200/O2000

- Ciprico 7000 RAID 3 Fibre Channel	:	~44 MB/sec

Ciprico FibreStore (JBOD)			:	>50 MB/sec

Clariion RAID 3 Fibre Channel		:	~38 MB/sec

Clariion JBOD FibreChannel		:	 ??

4 striped UltraSCSI drives			:	~30 MB/sec

- For O2

- UltraSCSI option drive			:	~ 7.0 MB/s

- Above rate’s are approximates and depend on the

stripe unit size which will be used

- Ciprico 7000 FC disk arrays are only ones used for testing

Bandwidth Considerations

- 8-bit 601 uncompressed     :  21 MB/sec

- RICE (DIVO only)             :  11 MB/sec

- DVCPRO                            :  4 MB/sec

- JPEG  				     :  1 MB/sec – 8 MB/sec

- MPEG-2                              :  3 Mb/sec – 8 Mb/sec

- Above rate’s are approximates

- RICE is a lossless coding algorithm thus few reasons to

use uncompressed

- Uncompressed currently not supported on O2

Required Software

- READ RELNOTES FOR UP-TO-DATE INFO!

- Overview of required software for the current VST

release 1.1

- Origin 200, Origin 2000 & Onyx2

- IRIX 6.4.1

- Recommended & Required patches Dec 1998

- Patches included on the VST CDROM

Optional: 	DIVO SW from VST CDROM

MediaPump SW from VST CDROM

- O2

- IRIX 6.5

- GRIO license is required if grio is used with

more then 4 simultaneous streams

- Plex license if plexing is used

##### Installing VST SW

- Install required patches, check relnotes for up-to-date

info about the required patches

Patches consist of

Recommended Required patches Dec 1998 or later

VST config specific patches from the VST CDROM

- Install eoe.sw.xlv

- Install eoe.sw.xfsrt

- Following VST subsystems are selected for installation

- by default

- vcp_recorder_eoe.man.base [d]

- vcp_recorder_eoe.man.relnotes [d]

- vcp_recorder_eoe.man.tools [d]

- vcp_recorder_eoe.sw.base [d]

- vcp_recorder_eoe.sw.tools [d]

- Make sure to install the appropriate VST subsystems

- required for media device support or other functionality

Installation of specific VST configs

- Origin 200/2000 & Onyx2 with DIVO-DVC

IRIX 6.4

Origin 200/2000 Required Patches for IRIX 6.4 (12/1/98 or later)

DIVO Video Execution Environment, 1.3 for IRIX 6.4

Patch 3249 (6.4 grio rollup #3; add FC support)

Patch 3249 is only required only for  a real-time fs Clip Cache

Patch 2505 (6.4 PCI Audio Expansion Card Support and audio fixes)

Patch 3416 (tserialio update for 6.4 (driver and user library))

Patch 3146 (dmedia Audio encode/decode and divo 1.3 on 6.4)

vcp_recorder_eoe.sw.divo (for VST DIVO media port support)

- Check following table for subsystems needed

for specific components such as:

Louth

Sony RS-422 control

VST enhanced vtrftpd

Etc.

VST subsystem				Description

vcp_recorder_eoe.books.AdminGuide	Installation and Administration

Guide

vcp_recorder_eoe.books.DevelGuide	Developers Guide

vcp_recorder_eoe.books.AdminGuideHTML	HTML Installation

and Admin Guide

vcp_recorder_eoe.books.DevelGuideHTML		HTML Developers Guide

vcp_recorder_eoe.man			Man pages

vcp_recorder_eoe.man.base			Base man pages

vcp_recorder_eoe.man.ftpd			Enhanced FTP daemon man pages

vcp_recorder_eoe.man.relnotes		Release notes

vcp_recorder_eoe.man.tools		Base applications ma n pages

vcp_recorder_eoe.sw.base			Base software

vcp_recorder_eoe.sw.diaquest		Diaquest deck-control module

vcp_recorder_eoe.sw.divo			DIVO video module

vcp_recorder_eoe.sw.dvb-vg		ViewGraphics DVB/MPEG module

vcp_recorder_eoe.sw.failsafe		Failsafe Configuration Files

vcp_recorder_eoe.sw.fsmon		File System Import Monitor

VST subsystem				Description

vcp_recorder_eoe.sw.ftpd			Enhanced FTP daemon

vcp_recorder_eoe.sw.horita			Horita TC input module

vcp_recorder_eoe.sw.little-red		Miranda Little Red TC input module

vcp_recorder_eoe.sw.louth			Louth VDCP control  module

vcp_recorder_eoe.sw.mpeg			MPEG2 tools

vcp_recorder_eoe.sw.o2video		O2Video module

vcp_recorder_eoe.sw.sony-odetics	Sony/Odetics control module

vcp_recorder_eoe.sw.studiocentral	StudioCentral archive interface

vcp_recorder_eoe.sw.tools			Base applications

vcp_recorder_eoe.sw.vela			Vela SCSI MPEG2- decoder module

vcp_recorder_eoe.sw.vlan			VLAN deck-control modul

- The /usr/vtr directories

bin/* 		utilities and VSTexecutable, vvtr

clips/* 		Clip Cache (can be symlink)

config/* 	Configuration files

data/* 		Files under this directory are static data or image files

used by various VSK components

images/* 	The directory under which still images are created

when the MVCP CIMG command is executed

index/* 		Clip indices (vframe formats, RICE, Uncomressed, JPEG)

install/ 		Files that are used during the installation

lib32/	 	Core and shared libraries

lib32/modules/ 	External interface device/control/format modules.

###### Setting up the Clip Cache

- Use of a real-time file is system strongly recommended

VST is designed to be used with real-time filesystems

enables the use of GRIO (disk I/O bandwith management)

enable features such as priority I/O for record/playout channels

- When using RAID, use RAID 3 only  (big I/O sizes)

- Use Ciprico FC RAID/JBOD disk arrays

- Disk space requirement guidelines

| Format | Rate MB/s | 1 min GB | 1 hour GB |
| --- | --- | --- | --- |
| Uncomp | 21 | 1.26 | 75.6 |
| RICE | 11 | 0.60 | 36.0 |
| DVCPRO | 4 | 0.24 | 14.4 |

- Striping guidelines for determining the stripe-unit

choice between:

disk bandwidth/memory consumption

latency

- USCSI disks 			256KB-512KB

- Fibre Channel JBOD		256KB-512KB

- Fibre Channel 4+1 RAID	1MB-2MB

- Fibre Channel 8+1 RAID	2MB-4MB

- The /usr/vtr/clips directory is the hard coded default

clip cache directory for VST

- The /usr/vtr/clips directory is the VST relative root

root  directory

/usr/vtr/clips/clip.rice		is clip.rice for VST

/usr/vtr/clips/fc/clip.dif	is fc/clip.dif for VST

- If multiple clip cache’s are required create

subdirs under /usr/vtr/clips

/usr/vtr/clips/rice

/usr/vtr/clips/rice-625

/usr/vtr/clips/mpeg

Don’t forget to create the required index directory

/usr/vtr/index/rice

/usr/vtr/index/rice-625

/usr/vtr/index/mpeg

- Use of symlinks is supported

Non-Real Time Clip Cache

- Options for setting up a non-real time Clip Cache

one option disk

several striped disks

one RAID 3 disk array

striped RAID 3 disk arrays (same as several striped disks)

- Memory page sizes:

32 bit kernels =  4  KB   (O2)

64 bit kernels = 16 KB   (Origin servers)

- The desired I/O size must be a multiple of the greater of the memory page size or filesystem block size

on a O200 that means that the I/O size needs to be a multiple of 16 KB

when 256KB-512KB stripe-unit sizes are used

this requirement will always be met

4 * 512 KB = 2048KB / 16KB = 128		valid I/O size

4 * 450 KB = 1800KB / 16KB = 112.5 	invalid I/O size

- The memory usage is determined by the I/O size of the

clip cache

- Always specify a blocksize of 16 KB when a filesystem

is created to be used as clip cache

- Example of creating a 3-way striped XLV using

USCSI option disks

xlv_make

xlv_make> vol clips

clips

xlv_make> data

clips.data

xlv_make> plex

clips.data.0

xlv_make> ve -stripe_unit 1024  dks2d1s7 dks2d2s7 dks2d3s7

clips.data.0.0

xlv_make> end

Object specification completed

xlv_make> exit

Newly created objects will be written to disk.

Is this what you want?(yes)

Invoking xlv_assemble

NOTE: the stripe_unit is in disk blocks, 1 block = 512 bytes

- Next step is creating a filesystem

mkfs –b size=16k /dev/xlv/striped

- Mount XLV volume on /usr/vtr/clips

- Symlinks are allowed

- Put corresponding entry in /etc/fstab

- Instruct VST to use a I/O size equal to stripe size

of the created XLV

using the stripe unit used in the above example the I/O needs to be:

512 KB * 3 disks  = 1536 KB

create the /usr/vtr/config/device-defaults/vtrfsinfo.conf file

add /dev/xlv/striped	1536K 	1536K

See “Configuring VST”  for detailed info on vtrfsinfo.conf

- Memory usage per channel is 3 * I/O size. Double this

When seamless back-to-back is required

- In case of a single RAID 3 array

create a filesystem with a 16 KB blocksize

create the vtrfsinfo.conf file

- Treat a striped RAID XLV the same way as striped disks

- In case of a single disk create a a filesystem with a

16KB blocksize

- Modify the maxdmasz if the required I/O size is larger then 4 MB

see configuring SVT

Real-Time Clip Cache

- Real-Time XLV guidelines

data subvolume should be 200 MB

stripe unit between 256KB-512KB

try to put only real-time volumes on controller

when using RAID, the use of two internal disks to plex the

data subvolume is advisable (single-point-failure)

- Options for setting up a non-real time Clip Cache

several striped disks

one RAID 3 disk array

striped RAID 3 disk arrays (same as several striped)

seperate option disk is required for the data subvolume

or use the system disk, this requires repartion and reinstallation of OS

- The desired I/O size must be an multiple of the

greaterof the memory page size or fs block size

on a O200 that means that the I/O size needs to be a multiple of 16 KB

when 256KB-512KB stripe-unit sizes are used

this requirement will always be met

4 * 512 KB = 2048KB / 16KB = 128		valid I/O size

4 * 450 KB = 1800KB / 16KB = 112.5 	invalid I/O size

- Example of creating a XLV for a real-time filesystem

which will be striped across 2 RAID units

xlv_make

xlv_make> vol rt_clips

rt_clips

xlv_make> data

rt_clips.data

xlv_make> plex

rt_clips.data.0

xlv_make> ve dks0d2s7

rt_clips.data.0.0

xlv_make> rt

rt_clips.rt

xlv_make> plex

rt_clips.rt.0

xlv_make> ve -stripe_unit 4096 dks2d1s7 dks2d2s7

rt_clips.rt.0.0

xlv_make> end

Object specification completed

xlv_make> exit

Newly created objects will be written to disk.

Is this what you want?(yes)

NOTE: the stripe_unit is in disk blocks, 1 block = 512 bytes

the extentsize needs to be equal to the stripe size

for  this example that is

4096 * 512 * 2 = 4194304 bytes = 4096 KB = 4 MB

In this example the memory usage will be 4 MB * 3 = 12 MB

- Create a real-time filesystem

use a blocksize of 16 KB (always!)

use a extensize equal to the stripe size

for the above example this would be

mkfs –b size=16k –r extsize=4m /dev/xlv/rt_clips

- Mount fs on /usr/vtr/clips

- Add a /etc/fstab entry

- Configuration of the vtrfsinfo.conf is not needed, VST

will automatically us the correct I/O size

- If a different I/O size is required, don’t recreate the fs. Just add an entry in the vtrfsinfo.conf with the required

I/O size

- Modify the maxdmasz if the required I/O size is larger then 4 MB

see configuring SVT

###### Configuring GRIO

Striped XLV

- First step: /etc/init.d/grio start

ggd will report unkown disks

- When there are any unkown disks, run grio_bandwidth

to determine OPS for desired I/O size

- In case of striped disks use stripe unit as I/O size

Run xlv_assemble to determine the stripe unit of the disks

xlv_assemble: Checking for Disk Plexing Option ... done

…

start=0, end=71116799, (stripe)grp_size=2, stripe_unit_size=1024

/dev/dsk/dks0d6s7 (35558944 blks)

/dev/dsk/dks0d5s7 (35558944 blks)

…

In this example the stripe_unit is 1024 DISK BLOCKS = 512KB

grio_bandwidth –d /dev/dsk/dks0d6s7 –t 60 –s 524288 -u

-d	disk device (use the disk device grio reported as unkown)

-t	number of seconds the test is performed (60 sec. is min)

-s	I/O size in BYTES

-u	update /etc/grio_disks automatically

- Re-run /etc/init.d/grio start, when the striped volume

consists of only 1 type of disk grio will report the

number useable streams (4 without GRIO license)

- If the striped volume consist of several types of

disks grio will report the following unkown disk

Repeat grio_bandwidth for all the disk untill grio

doesn’t report any error messages anymore

Real-time filesystem covering one partion

- If one FC RAID 3 disk array is used the OPS for

the extensize of the real-time fs needs to be determined

Example:

Ciprico 7000 with a real-time filesystem  that has a 4 MB extentsize

use xfs_growfs –n /mountpoint to determine extentsize

xfs_growfs -n /usr/vtr/clips

meta-data=/usr/vtr/clips isize=256	      agcount=8, agsize=8192 blks

data     = 		       bsize=16384      blocks=65536, imaxpct=25

log      =internal	       bsize=16384      blocks=1000

realtime =external	       bsize=4194304  blocks=8890880, rtextents=34730

The last line reports the extentsize in this case it is 4194304 bytes = 4 MB

1024 * 1024 * 4 = 4194304 bytes = 4096 KB = 4 MB

- Run grio_bandwith, specify extensize as I/O size

grio_bandwith –d /dev/dsk/dks2d1s7 –t 60 –s 4194304

System will reports something similar to this:

Device /hw/module/1/slot/io7/MotherBoard/pci/3/scsi_ctlr/0/target/1/lun/0/disk

with ID “Ciprico Rimfire 7010 03.1”,

using only read ops, provided:

on average: 10 ops of size 4194304 each second

in the worst case: 9 ops of size 4194304 each second

- Use the /hw path obtained from grio_bandwidth to

edit /etc/grio_disks file

REPLACE /hw/module/1/slot/io7/MotherBoard/pci/3/scsi_ctlr/0

4194304 25

REPLACE

/hw/module/1/slot/io7/MotherBoard/pci/3/scsi_ctlr/0/target/1/lun/0/disk

4194304 11

NOTE: Each REPLACE entry should be placed on one line!

- Restart grio, /etc/init.d/grio start

GRIO should report numbers of useable streams

- When several RAID units have been used to create a striped

XLV use the stipe unit as I/O size instead of the extentsize

- When JBOD is used determine the OPS for one

disk and put a REPLACE statement for each disk

in the /etc/grio_disk file

REPLACE /hw/module/1/slot/io3/fibre_channel/pci/1/scsi_ctlr/0 524288 200

REPLACE /hw/module/1/slot/io3/fibre_channel/pci/1/scsi_ctlr/0/target/1/lun/0/disk 524288 25

REPLACE /hw/module/1/slot/io3/fibre_channel/pci/1/scsi_ctlr/0/target/2/lun/0/disk 524288 25

REPLACE /hw/module/1/slot/io3/fibre_channel/pci/1/scsi_ctlr/0/target/3/lun/0/disk 524288 25

REPLACE /hw/module/1/slot/io3/fibre_channel/pci/1/scsi_ctlr/0/target/4/lun/0/disk 524288 25

REPLACE /hw/module/1/slot/io3/fibre_channel/pci/1/scsi_ctlr/0/target/5/lun/0/disk 524288 25

REPLACE /hw/module/1/slot/io3/fibre_channel/pci/1/scsi_ctlr/0/target/6/lun/0/disk 524288 25

REPLACE /hw/module/1/slot/io3/fibre_channel/pci/1/scsi_ctlr/0/target/7/lun/0/disk 524288 25

REPLACE /hw/module/1/slot/io3/fibre_channel/pci/1/scsi_ctlr/0/target/8/lun/0/disk 524288 25

- When GRIO (ggd) is running check the bandwidth

for the configured real-time filesystem

grio -C -d /dev/dsk/dks3d1s7

GRIO information for path of disk device /dev/dsk/dks2d2s7

GRIO information for device:

/hw/module/1/slot/io7/xbox_dualxtown/pci/0/scsi_ctlr/0/target/2/lun/0/disk

opt i/o size: 4194304 bytes

reservations: 1 ops currently reserved (max. per quantum), 10 ops maximum

bandwidth: 36864 kbytes/sec available, 40960 kbytes/sec maximum

GRIO information for device:

/hw/module/1/slot/io7/xbox_dualxtown/pci/0/scsi_ctlr/0

opt i/o size: 4194304 bytes

reservations: 1 ops currently reserved (max. per quantum), 25 ops maximum

bandwidth: 98304 kbytes/sec available, 102400 kbytes/sec

Clip Cache for Uncompressed video

- RICE is lossless coding, thus few reasons to use

uncompressed

- If uncompressed is a requirement the following guidelines will achieve the best performance

- NTSC (525) Uncompressed Clip Cache

Stripe unit of 552 disk blocks = 226 KB

Stripe size of 2260992 bytes for 8 striped disks

Stipe size holds exactly 6 uncompressed 4:2:2 8-bit fields

Stripe size of 1081344 bytes for 4 striped disks

Stipe size holds exactly 3 uncompressed 4:2:2 8-bit fields

- PAL (625) Uncompressed Clip Cache

Stripe unit of 624 disk blocks = 312 KB

Stripe size of 2654208 bytes for 8 striped disks

Stripe size holds exactly 6 uncompressed 4:2:2 8-bit fields

Stripe size of 1327104 bytes for 4 striped disks

Stipe size holds exactly 3 uncompressed 4:2:2 8-bit fields

VST Configuration

- The following topics will be discussed in this module

### VST config directory

Overview of the structure of the VST config directory.

All config files will be discussed

### Configuration of the optional VST components

Discussion of how to configure components which can be installed optionally

### VST config directory

- There are two ways of configuring VST

config files 	read during startup (static)

via MVCP 	dynamic

- This part covers the static config files

- All config files are stored in /usr/vtr/config

- The files in this directory will *only* be read at startup (vtrstart)

- Structure of /usr/vtr/config

/usr/vtr/config				for general config files

/usr/vtr/config/device-defaults	device controls

/usr/vtr/config/system-defaults	system controls

- system controls can only be set during startup

- device controls can be set during and after startup

# General config files

- These files are used to configure specific components

(e.g., vtrftpd), the vtr deamon and control protocol

devices (e.g., Louth, Sony)

- Files in the /usr/vtr/config directory:

control-in.conf

control-out.conf

ports.conf

studiocentral.conf

vtrd.conf

vtr_ioconfig.conf

vtrfsinfo.conf

vtrftpd.conf

- Not all of the above config files will be created

automatically

#### The control-in.conf file

This file is used to configure devices which control VST

The currently supported remote devices are those supporting the  protocols defined by Sony, Louth and Odetics

- The control-in.conf file has three parts:

Network transmission specification	one line per file

Serial port configuration			one line per device

Signal configuration				one line per device

- sample control-in.conf

mvcp tcp 5250

louth		rs422		2	38400	1	1	29.97	3

louth		rs232		3	38400	1	2	29.97	3

louth		tcp	   8777	-		-	3	25		3

odetics	rs422		4	38400	1	4	29.97	0

sony		rs422		5	38400	1	5	29.97	0

p2		rs422		6	38400	1	6	30		0

hsip		rs232		8	9600		0	1	29.97

signal  1  DIVO_0

signal  2  DIVO_1

signal  3  DIVO_2

signal  4  DIVO_DVC_4

signal  5  DIVO_3

signal  6  DIVO_DVC_1

- Network transmission specification line

this line is used to configure and enable MVCP

syntax:

protocol_name transmission_medium port_number

only valid line:

mvcp  tcp  5250

if tcp port 5250 is in use by another application a different tcp

port number can be used.

this line is required to use MVCP

when this line is missing or a different tcp port number is used

the VST GUI’s won’t work

- Serial Port Configuration Lines

these lines are used to configure the serial controlling devices

such as Sony, Louth and Odetics

syntax

<prot> <comm type> <comm port> <speed> <parity> <video> <rate> <preroll>

example:

sony rs422 4 38400 1 1 29.97 0

a remote device is controling a media port which is associated with

signal 1 using sony protocol and a framerate of 29.97.

in this case it connected to serial port 4 which is to 422 mode,

38400 bps and odd parity. Pre-roll is 0

<prot>

The <prot> field specifies control protocol wich will be used

Valid values are

tcp

sony

p2

odetics

louth

vlan

hsip (Horita)    (timecode reader)

littlered            (timecode reader)

to use sony/p2/odetics install	vcp_recorder_eoe.sw.sony-odetics

to use louth  install			vcp_recorder_eoe.sw.louth

to use hisp install			vcp_recorder_eoe.sw.horita

to use littlered install			vcp_recorder_eoe.sw.little-red

Note: P2 is the Panasonic name for the Sony RS-422 protocol

<comm type>

The <comm type> field determines in which mode in the serial port of the server will be controlled

Valid values are

rs232, for a RS-232 control connection.

rs422, for a RS-422 control connection.

<comm port>

The value of  <comm port> must be an integer that corresponds to the existing serial port on the server

### A value of 1 is not supported. This is the console port

<speed>

The <speed> value specifies the connection speed between the serial port on the server and the controlling device

Valid values are

9600

14400

28800

38400

note: for Sony,  P2 and Odetics devices the value is always 38400

<parity>

This field determines the parity of the connection

Valid values are

0	indicating no parity

1	indicating odd parity

2	indicating even parity

odd parity is used by most control protocols

no parity is the setting required for timecode readers

<video>

The <video> value is an arbitrary unique number that corresponds to the signal number associated with a media port

In case of a TC reader this value is the time channel

This means that you can have two serial config lines with the same video number

p2		rs422		6	38400	1	1	30		0

hsip		rs232		8	9600		0	1	29.97

A value of 0 is not supported

<rate>

The <rate> value specifies the number of frames per second displayed. Also referred to as framerate

Valid values are

24		for film

25		for the PAL video format

29.97	for NTSC drop-frame video

30		for NTSC non-drop-frame video

<preroll>

This field determines the pre-roll of a issued command.

The pre-roll is only required for Louth and is used to

compensate the latency of the media port.

The pre-roll value depends on the media port

use a value of 3 for Louth 	->	DIVO

use a value of 5 for Louth	->	DIVO-DVC

use a value of 0 for all other protocols

control-out.conf file

this file is used to configure VST as a controlling device.

currently supported outbound control protocols are

Sony RS-422 and VLAN

syntax

<prot> <comm type> <comm port> <speed> <parity>

example

vlan       rs422   2   38400     1

sony      rs422   3    38400    1

to use vlan install vcp_recorder_eoe.sw.vlan

to use sony install vcp_recorder_eoe.sw.diaquest

<prot>

The <prot> field specifies the control protocol to use

The valid values are

sony

p2

vlan

to use sony install		vcp_recorder_eoe.sw.diaquest

to use vlan				vcp_recorder_eoe.sw.vlan

<comm type>

The <comm type> field determines in which mode in the serial port of the server will be controlled

Valid values are

rs232, for a RS-232 control connection.

rs422, for a RS-422 control connection.

<comm port>

The value of  <comm port> must be an integer that corresponds to the existing serial port on the server

### A value of 1 is not supported. This is the console port

<speed>

The <speed> value specifies the connection speed between the serial port on the server and the controlling device

Valid values are

9600

14400

28800

38400

note: for Sony,  P2 and Odetics devices this value is always 38400

<parity>

This field determines the parity of the connection

Valid values are

0	indicating no parity

1	indicating odd parity

2	indicating even parity

odd parity is used by most control protocols

no parity is the setting required for timecode readers

#### The ports.conf file

This file can be used the to map physical port names such as DIVO_0, to logical port names, such as ChA

example

port DIVO_0 ChA “On-Air Channel A”

port DIVO_1 ChB “On-Air Channel B”

vtrstat -p

#  Port    Type    Description

----------------------------------------------------------

1  ChA    Video   On-Air Channel A

2  ChB    Video   On-Air Channel B

#### The vtrd.conf file

This is the configuration file to set options for the SVT core executable vvtr

syntax

name cpu# priority min-retry(ms) max-retry(ms) term-signal command

example

vvtr * * * * TERM “/usr/vtr/bin/vvtr –f2”

normally you don’t edit this unless

some debugging is required			(add –f2)

the number of required units > 32		(add –u64)

The vtr_ioconfig.conf file

This file is used to remap the vela logical numbers.

This file has the same functionality as the

/etc/ioconfig.file (see man ioconfig)

This file won’t be created by default.

#### The vtrfsinfo.conf file

This file is used to set the I/O and alignment size for

non-real time filesystems or change the I/O and alignment size for real-time filesystems

This file needs to be  created manually

syntax

device_name     I/O_size   alignment_size

example

/dev/xlv/striped 	4m 	4m

device_name

This is the device path for the disk or the XLV volume

Valid device paths are

/dev/dsk/dks2d1s7

/dev/xlv/striped

### I/O_size

This is the desired I/O size for the none real-time fs

In case of a striped XLV volume this value should be equal to the stripe size

some valid (integer) values are

4096k

2m

alignment_size

Should always be equal to the I/O_size. Unless the clips

recorded will be transferred to a system with a different alignment size.

Some valid (integer) values are

1m

1536k

note: 	if  an I/O size larger then 4 MB will be used modification

of the maxdmasz kernel variable is required!

#### The vtrftpd.conf file

This file is used to determine the bandwidth which will be allocated when a clip is imported to the real-time filesystem

syntax

pathname_pattern     bandwidth

example:

/usr/vtr/clips/*          28800000

each file put transferred  the /usr/vtr/clips directory will be put in

there with a resereved fixed rate of  28.8 Mb/s

pathname_pattern

This value specifies which clips stored on a real-time filesystem will be guaranteed a specified bandwith while storing. When there’s no match the default bandwidth allocation specified in /etc/inetd.conf will be used

vtrftpd -l -G30000000 -P126 	(the –G option)

bandwidth

This value specifies how much bandwith (in bits) will be used to store the clip when the pathname_pattern matches

### System controls

- System controls are configuration variables not associated with specific devices but used for general system behaviour, e.g. time, grio

- Can only be set during startup

- Config files in this directory are named according to the name of the subsystem

vtr.time.slave_system_time

would be put into /usr/vtr/config/system-controls/time

- examples of files in system-defaults/

fs

main

time

sony_1

these files are not created by default

- Check man vcp-recorder-controls for currently supported device controls and their options

- Creation or modification of these files is normally

not needed and not encouraged

# Device Controls

- Device controls are configuration, control and status variables associated with specific device interface or unit instance

- Can be set during startup and set or queried through the MVCP SET & GET commands

- Config files in this directory are named according to the name of the logical media port name (vtrstat –p) and are case sensitive

- examples of files in system-defaults/

mvp

DQ_1

DIVO_DVC_1

dvb-asi

vela_24

these files are not created by default

- Check man vcp-recorder-controls for existing

device contols and values

- Creation or modification of these files is normally

not needed and not encouraged

- Unit devices are initialized according to the following order

settings for all devices:	ALL

settings for node type:		MEDIA or STORAGE

settings for port type:		VIDEO, NETWORK, DECK, or DISK

settings for device class:	<device-class>

settings for device:		<device-name>

For example, when the unit device interface for the DIVO_DVC_ 4  video device is initialized, settings are loaded from the

device-defaults files as follows

ALL

MEDIA

VIDEO

DIVO_DVC

DIVO_DVC_4

example

DIVO_DVC

vtr.media.audio.input.port ““

DIVO_DVC_4

vtr.media.audio.input.port “RAD1.AESIn.AESInput”

result

all audio input ports of the DIVO_DVC media ports will be disabled except the DIVO_DVC_4 media port, which will be enabled and take input from  the AES Input of RAD 1

# Configuration of maxdmasz

- To allow VST to use large I/O operations the

maxdmasz kernel variable needs to be adjusted

default maxdmasz value: 	1025 (32-bit Kernels) = 4MB

257   (64-bit Kernels) = 4MB

the value is the # of pages

the page size is 4K for	    32 bit kernels  (O2)

the page size is 16K for  64 bit kernels  (Origin)

example

VST on a O200 needs to be able to do 16 MB I/O operations

16 MB = 16 * 1024 = 16386 KB

maxdmasz = I/O / page size = 16386 / 16 = 1024 pages

systune -i

updates will be made to running system and /unix.instal

systune-> maxdmasz 1024

maxdmasz = 257 (0x101)

do you really want to change maxdmasz to 1024 (0x400) ?

(y/n) yes

in order for the change in parameter maxdmasz to become

effective, reboot the system

systune-> quit

autoconfig

reboot

# Configuring Audio

- Audio configuration is only supported for

the  DIVO & O2 Video media devices

- Two options to configure audio

via the media port specific device config file

via the MVCP SET command

- Configuration via the media specific device file config

file in /usr/vtr/config/device-defaults dir can be

DIVO

DIVO_3

DIVO_DVC

DIVO_DVC_1

mvp

- Disabling audio

When no audio is required the audio can be disabled by the following controls

vtr.media.audio.input.port ""

vtr.media.audio.output.port ""

- The device controls to use

vtr.media.audio.input.port

vtr.media.audio.autoput.port

default: DefaultIn

DefaultIn is the default input port configureable via apanel. If set to VideoIn, audio input is taken from the digital audio embedded in the digital video signal (SMPTE 272M)

the port may also be specified using the form:

[<subsystem>.]<device>.<interface>

incase of Radical audio subsystem

subsystem is "RAD<n>"

device is one of "AnalogIn", "AESIn", or "ADATIn"

interface is

"LineIn" or "Microphone" for "AnalogIn"

"AESInput" or "OpticalIn" for "AESIn"

example

to take audio input for the DIVO_1 video port from the AES coaxial input on a Graphics BaseIO, edit the file ../device-defaults/DIVO_1, and add this control setting:

vtr.media.audio.input.port "RAD1.AESIn.AESInput"

### Configuration of the optional VST components

# Installing Time Code Readers

- Timecode readers set internal “wall-clock” time

- When a timecode reader is installed the IRIX time

will be slaved to the timecode source by default

- VST supports multiple time channels

- Two types of  time code readers are supported

Horita

Miranda Little Red

- Required subsystems

vcp_recorder_eoe.sw.horita		for the Horita TC reader

vcp_recorder_eoe.sw.little-red	for Miranda Little Red TC reader

- To setup TC readers, edit control-in.conf file

hsip         rs232  2 9600 0 1 29.97

little-red  rs232  3 9600 0 2 25

syntax to use is explained in the control-in.conf section

- The <video> field is in case of TC readers the time_channel

# Configuring Sony Protocol

- Enables VST to be controlled by a VTR (slave mode)

- P2 is alternate name for Sony Protocol

- Sony Protocol is not supported on a O2

- Sony protocol is partially supported by VST

- Unsupported Sony Commands

In the current VST release they fall into several categories:

video output parameter selection

audio output parameter selection

audio split editing

insert editing for anything less then extant video/audio tracks

+/-15% playback/recording/editing  (this is not the same as

VARIABLE PLAY mode, which is fully implemented)

note: the O2 does not support deck emulation

- there are two steps to configure Sony protocol

create /usr/vtr/config/system-defaults/sony file

modify  /usr/vtr/config/control-in.conf

- The controls supported by the Sony control module

vtr.control.device_type_id

vtr.control.output.idle_mode

vtr.control.timecode.mode

vtr.control.clip.name

vtr.control.superimpose.enabled

vtr.control.edit.delay

vtr.control.ee.delay

vtr.control.ee.mode

vtr.control.ee.record_select

vtr.edit.preroll

vtr.edit.postroll

these can be put in the ../system-default/sony_x file or be set and queried through the MVCP SSET and SGET commands

- Create a sony protocol entry the control-in.conf file

sony  rs422  2  38400  1  1  29.97  0

signal 1 DIVO_0

- Connect the VTR RS-422 port to the desired serial port of the Origin

- See VST installation guide for cable to use

- Two logical units are allocated for each configured Sony port. One record & one play unit

- DO NOT CONNECT TO MEDIA PORT OR OPEN A UNIT OWNED BY A SONY PORT

- Via the mclips GUI the loaded clip can be changed

# VST GUI’s

- VST includes the following GUI’s

mcpanel		media control panel (virtuel VTR panel)

deck control	remote VTR control

mcclips		clip management

mcstat		unit status monitor

- The GUI’s are designed as a demonstration tool and

a beginning point from which broader graphical applications can be developed

- VST is not ment to be used as an application but can be used out-of –the-box with the included GUI’s to give a demonstartion

- This section is a brief overview of the GUI’s.

- The GUI’s are very inituitive and well documented in the VST Developers Guide

- All GUI’s use MVCP commands to control VST

- To see which MVCP commands are issued use the

-v 2 option when the GUI is started

mcpanel

- The Media Control Panel is a virtual VTR control panel

- When mcpanel is started a connection can be made to a media port or to a existing unit

/usr/vtr/bin/mcpanel hostname videoPort|unit

- mcpanel has the following start options

/usr/vtr/bin/mcpanel [-h hostname][-p videoPort|unit]

[-c clipname]

[-r]

[-C "inpoint outpoint"]

[-P]

[-v loglevel

[-D deckCtlPort]

-c 	Identifies the name of a clip to be loaded when the Media

Control Panel starts. If this option is not specified, no clip is	initially loaded.

-D	Specifies the video deck control port (deckCtlPort) to be used

for V-LAN communication to an external video storage device, such as a digital videotape recorder (VTR). The deck

is controlled by the Deck Control Panel. If this option is not specified, deck control is not available

-h 	Identifies the host on which VCP-Recorder runs. This enables the Media Control Panel to be run from a remote

workstation. If this option is not specified, the local host is assumed

-p 	Identifies the VCP-Recorder host video port or unit to which

he control connectio is made (check with vtrstat –p)

-r 	Specifies that if an mcpanel already exists for the video port, the existing mcpanel should be raised on the desktop instead of creating a new one

-C 	Specifies that the loaded clip should be cued with the specified in- and out-points If “*” is specified for either inpoint or outpoint, the default edit in-point or out-point is used

-P 	Specifies that the clip whose name is clipname should start

playing when the Media Control Panel starts

-v 	Sets the logging verbosity level to loglevel

- mcpanel can be started from any workstation and connect to any networked VST server using the

–h hostname option

- When mcpanel is started on the local VST server, the default behaviour is that there will be unit created on the media port with the lowest logical number

vtrstat  -p

#  Port    Type    Description

----------------------------------------------------------

1  DIVO_DVC_0 Video   SGI Digital Video Option (DVC)

2  DIVO_DVC_1 Video   SGI Digital Video Option (DVC)

In this example that will be DIVO_DVC_0

- The options of  the direction selector

Fwd		for forward play (default)

F Lp		for forward loop play

Bwd		for backward play

BLp		for backward loop play

F/Bwd	for alternating forward and backward play

F/B Lp	for alternating forward and backward loop play

B/Fwd	for alternating backward and forward play

B/F Lp	for alternating backward and forward loop play

- The playbackmode options are

PB		the clip’s audio and video are output when a clip is

playing; nothing is output at other times (default)

PB/EE	the clip’s audio and video are output when a clip is

playing; the input signal is output at other times

Note: The EE (end-to-end) option emulates a video deck feature

and is relevant only when you have an active input source

PB/Im	the clip’s audio and video are output when a clip is playing; SMPTE 75% colors bars and 1 kHz tone are output at other times

PB/B	the clip’s audio and video are output when a clip is

playing; a black screen is output at other times

EE		the output always displays the input signal instead of

the signal from VST, even when a clip is playing

(When a clip is playing, the output displays the input signal)

Image	the output always displays SMPTE 75% colors bars and plays a 1 kHz tone

Black	the output always displays a black screen

Hold		the output always displays the last image

- Procedure for playing out clips

connect a mcpanel to the desired media port or unit

load the clip

determine the direction

cue the clip

play the clip

- When recording clips make sure to select the right format and compression

- Start the Deck Control window from the mcpanel

connected to the media port which is connected to the VTR

- When the V-LAN or the diaquest media port module

is installed the remote VTR can be controlled via the

the deck control GUI.

- See Developers Guide for details how to use this option

mcclips

- mcclips is the VST clip mager with basic file management  fuctions as

renaming

deleting

setting the protections

archive interoperability

- Like the other GUI’s a remote VST server can be managed by using the –h hostname option

/usr/vtr/bin/mcclips [-v loglevel][hostname]