NetWare Driver Installation             December 1992

===========================

This diskette is labeled as LAN_DRV_DLK and the files are organized as follows:

Directory             Contents

---------             --------------------------------------------------------

\NETWARE              A pre-generated IPXDE150.COM

\NETWARE\V2XX         NetWare v2.15b, v2.15c or v2.2 server & bridge drivers

\NETWARE\V215B        NetWare v2.15b workstation drivers

\NETWARE\V215C        NetWare v2.15c, v2.2 or NetWare 386 workstation drivers

\NETWARE\ODI.DOS      ODI workstation driver for DOS

*********************************

*  QUICK SETUP FOR WORKSTATION  *

*********************************

A pre-generated workstation driver IPXDE150.COM is included in this disk.

You can simply run the IPXDE150 on a workstation to access the Novell network:

>IPXDE150    --Supports NetWare Lite, 2.1x, 2.2 or NetWare 386

>NETx        --This program is provided by Novell

***********************

*  WORKSTATION SETUP  *

***********************

If you want to generate your own workstation driver instead of using the

provided IPXDE150.COM, please do the following:

1)  If you make a backup of this diskette, make sure that it is labeled as

LAN_DRV_DLK.  You can use the DOS "Label" command as follows:

>Label  A:LAN_DRV_DLK

2)  Copy the appropriate NetWare driver files to the root directory of

this diskette (or backup):

>Copy  A:\NETWARE\V215c\*.*  A:\   --For NetWare 386 workstation

3)  Execute Novell's SHGEN or WSGEN program, which resides in the SHGEN-1 or

WSGEN diskette.  When you are asked to select a network driver, press

the INS key.  You will be asked to insert a LAN_DRV_??? diskette.  Insert

this diskette (or backup) in drive A.  Upon completion, a new IPX.COM file

will be created in your SHGEN-2 or WSGEN diskette.

Note : if you generate IPX.COM by "1:Driver Configurable by JUMPERS Utility"

the I/O PORT # will be None, then you must execute JUMPERS Utility

offered by NETWARE, the procedure as follow:

a. >JUMPERS

b. Enter the file "IPX.COM" to modify

c. Screen display as follow:

DE-150 Options

I/O PORT #1     None

d. Press Enter

(window assistance)

I/O PORT#

=========

200

210

230

240

.

.

.

e. Select one port#

f. Save the port# that you selected.

************************

*  NETWARE LITE USERS  *

************************

1)  Follow the normal installation procedures as instructed by NetWare Lite.

When asked to select a network driver, simply choose "NOVELL NE2000".

2)  After installation completed, modify the STARTNET.BAT file as follows:

From:

---------------------------

LSL

NE2000

IPXODI A

CLIENT

To:

---------------------------

LSL

DE150ODI

IPXODI A

CLIENT

3) Copy \NETWARE\ODI.DOS\NET.CFG file to NWLITE sub-directory which was

created when you installed NetWare Lite.

4) You may run STARTNET to access the Novell network.

Example 2:  NetWare v2.2 Server/Bridge Setup

============================================

1)  Copy the appropriate NetWare driver files (from this diskette) to the

root directory of the diskettes labeled SYSTEM-1 and LAN_DRV_DLK.

>Copy A:\NETWARE\V2XX\*.LAN  B:\    --SYSTEM-1 disk must be in drv B

>Copy A:\NETWARE\V2XX\*.OBJ  A:\    --Copy the files to the root dir

2)  Execute Novell's INSTALL program, which resides in the SYSTEM-1 diskette.

Choose the "D-Link Ethernet DE-150" driver when asked for a network driver.

Example 3:  NetWare v2.15b Server/Brige Setup

=============================================

1)  Copy the appropriate NetWare driver files (from this diskette) to the

root directory of your Novell diskette labeled LAN_DRV_DLK.

>Copy  A:\NETWARE\V2XX\*.LAN  B:\ --AUXGEN disk must be in drive B

>Copy  A:\NETWARE\V2XX\*.OBJ  A:\ --LAN_DRV_DLK disk must be in drive A

2)  Execute Novell's NETGEN program, which resides in the NETGEN diskette.

Choose the "D-Link Ethernet DE-150" driver, when asked for a network

driver.

************************************

* Install NetWare V2.2 workstation *

************************************

Refer to procedure  A. HOW TO SETUP IPX and follow all procedures

except for #3.  Instead of running the program SHGEN.EXE, run WSGEN.EXE .

**********************

*  ODI DRIVER SETUP  *

**********************

ODI Workstation for DOS

=======================

D-Link ODI Driver program allows you to run multiple protocl stacks.  It also

allows you to install up to two network adapters in a workstation. D-Link ODI

driver supports "port, node address, protocol and frame" options.  You can use

these options to create your own NET.CFG file.  For detailed info on Keywords,

please refer to Novell manual "NetWare ODI shell for DOS - NET.CFG options".

1)  Copy the DE150ODI.COM file (from this diskette) to the directory which

contains Novell's LSL.COM, IPXODI.COM and NETx.COM files.

2)  Follow Novell's instructions on using the ODI programs. You can substitute

the DE150ODI.COM whenever a reference is made to the Hardware Specific

Module.

3)  A typical startup for ODI workstation is as follows:

>LSL               --Link Support Layer Module provided by Novell

>DE150ODI          --Hardware Specific Module provided by D-Link

>IPXODI            --IPX Protocol Stack Module provided by Novell

>NETx              --DOS Support Module provided by Novell

>LOGIN