December 1992

D-Link DE-150 8-Bit Ethernet Card

FTP PC/TCP Installation

=================================

The following files will be used in this installation:

Files           Contents

-----------     -------------------------

DE150PD.SYS     DE-150 Driver Program

DE150PD.COM     DE-150 Driver Program

The D-Link driver programs above let you run FTP's PC/TCP

with the D-Link DE-150 Ethernet cards.  Two methods can be

used:

METHOD 1:

If you use the DE150PD.SYS program, the following statement must

be included in your CONFIG.SYS file:

DEVICE = DE150PD.SYS <packet_int_no> <int_level> <mem_base> <io_addr>

This statement may be placed anywhere in the CONFIG.SYS file.

METHOD 2:

If you use the DE150PD.COM program, follow the procedure below:

1. Execute the DE150PD.COM as follows:

>DE150PD <packet_int_no> <int_level> <mem_base> <io_addr>

2. Execute the ETHDRV.EXE device driver program (provided by FTP).

You are now ready to use FTP's PC/TCP.

Notes:

<packet_int_no> is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

<int_level>     is the hardware interrupt (IRQ) line set on your network

interface card. The default is 3 if this parameter is not

specified.

<mem_base>      is base memory address set on your network interface card.

The default address is D000H, if this parameter is not

specified.

<io_base>       is base I/O address set on your network interface card.

The default address of 300H is used, if this parameter

is not specified.