December 1992

D-Link DE-150 8-Bit Ethernet Card

PC-NFS Driver Installation

=================================

The D-Link PC-NFS Driver Program lets you run SUN Micro Systems PC-NFS

on your PC with the D-Link DE-150 8-Bit Ethernet Card.

The following files will be used in this installation:

Files           Contents

-----------     -------------------------

PDNFS.SYS       PC-NFS Driver Program

DE150PD.SYS     DE-150 Driver Program

DE150PD.COM     DE-150 Driver Program

Two methods can be used to install:

METHOD 1:

1. Install the PC-NFS program according to the procedure

described in the PC-NFS manual.  On the screen of the PC-NFS

install program, select 3C501 (the 3Com EtherLink card) as your

network interface card.  When the screens prompt you, select 3

for the interrupt level (IRQ), and select 300h for the base I/O

address.

2. Copy the following files to the \NFS directory of your hard

disk:

PDNFS.SYS

DE150PD.SYS

DE150PD.COM

4. Modify the CONFIG.SYS file as follows:

DEVICE=C:\ANSI.SYS

DEVICE=C:\NFS\PCNFS.SYS

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE=c:\NFS\DE150PD.SYS <packet_int_no> <int_level> <mem_base> <io_addr>

DEVICE=C:\NFS\PDNFS.SYS

LASTDRIVE=V

AUTOEXEC.BAT file:

SET PATH = C:\; C:\NFS

SET TZ=EST5EDT

NET YPDOMAIN ypserver

NET START RDR

NET YPSET homer

NET NAME john

NET USER F: \\user\john

We use the name "john" here as an example.

Notes:

<packet_int_no> is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

<int_level>     is the hardware interrupt (IRQ) line set on your network

interface card. The default is 3 if this parameter is not

specified.

<mem_base>      is base memory address set on your network interface card.

The default address is D000H, if this parameter is not

specified.

<io_base>       is base I/O address set on your network interface card.

The default address of 300H is used, if this parameter

is not specified.

5. Reboot the PC to start PC-NFS.

You may now execute the PC-NFS commands to communicate

other computers through the TCP/IP protocol.

METHOD 2:

1. Install the PC-NFS program according to the procedure

described in the PC-NFS manual.  On the screen of the PC-NFS

install program, select 3C501 (the 3Com EtherLink card) as your

network interface card.  When the screens prompt you, select 3

for the interrupt level (IRQ), and select 300h for the base I/O

address.

2. Copy the following files to the \NFS directory of your hard disk:

PDNFS.SYS

DE150PD.SYS

DE150PD.COM

4. Modify the CONFIG.SYS file as follows:

DEVICE=C:\ANSI.SYS

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE=C:\NFS\PCNFS.SYS

DEVICE=C:\NFS\PDNFS.SYS

LASTDRIVE=V

AUTOEXEC.BAT file:

SET PATH = C:\; C:\NFS

SET TZ=EST5EDT

DE150PD <packet_int_no> <int_level> <mem_base> <io_addr>

NET YPDOMAIN ypserver

NET START RDR

NET YPSET homer

NET NAME john

NET USER F: \\user\john

We use the name "john" here as an example.

5. Reboot the PC to start PC-NFS.

You may now execute the PC-NFS commands to communicate

other computers through the TCP/IP protocol.