December 1992

D-Link DE-150 8-Bit Ethernet Card

Test Program

=================================

The following files will be used to test your DE-150 card:

Files          Contents

-----------    -------------------------

TE150.EXE      DE-150 Test Program

DE-150.EXE     DE-150 Driver Program

The TE150.EXE program can be run to see if your DE-150 card has

any problems.  The procedure is as follows:

1. Copy DE-150 driver program DE-150.EXE into the directory where your

test program TE150.EXE resides.

2. Adjust switches of your DE-150 card to the proper position and insert

the DE-150 card in your computer. See your DE-150 card's Installation

Guide for details.

3. Execute the TE150.EXE program on your computer.

4. The test program will show the default jumper settings on the screen.

If your DE-150 card's jumper settings are different from default jumper

settings, change the jumper settings on the screen to match your

DE-150 card's jumper settings.

Enter a name for reference. This name is any name of up to 8

characters long, used to represent your computer.

Press RETURN.  The DE-150 card's driver program will be loaded in

from the current directory and the test begins.

5. Execute an appropriate D-Link test program on at least one

other computer on the network.  Example: if that computer uses a

DE-100 card, run TE100.EXE on that computer.  If that computer

uses a DE-150 card, run TE150.EXE on that computer.

6. If the network interface card is OK, the receive count will

steadily increase.

7. You can press F1 to reset receive count or ESC to terminate the test.

If the Test Program Fails

=========================

If test program fails, it does not necessarily mean that the

network interface card is defective. The failure may be due to

one or more jumper incorrect settings or to settings that pose

conflicts with other add-on cards present in your computer. The

failure may also be due to a defective trunk cable. Finally, the

failure may be caused simply by not inserting the network interface

card correctly in the computer.

The following lists the symptoms and their most likely causes.

Follow the remedies listed and retry the test program.

----------------------------------------------------------------

Symptom               Cause/Remedy

----------------------------------------------------------------

1.Different network   You have loaded a different driver program

interface program   before running the test program.

installed.

Reset your computer.

Re-run the test program.

2.Load execute file   The test program and its driver program are

name not found.     not at the same directory.

Copy the test program and its driver

program at the same directory.

Re-run the test program.