July 1992

LANsmart Driver Installation

============================

This installation procedure applies to the following D-Link products:

DE-200

DE-200TP

DE-200+

DE-200TP+

DE-205TP

The following files will be used in this installation:

Files        Contents

-----------  -------------------------------------------------

DE-200.DRV   Driver program for the above-mentioned adapters

DE-200.EXE   Driver program for the above-mentioned adapters

1. RUN LANsmart v.1.26 and v.1.30

==================================

Follow the procedure below to use the new LANsmart driver program on this

diskette with LANsmart v.1.26 and v.1.30.

1. Replace the LANsmart program LANMA05.DRV with the new driver program.

Example:

> COPY A:\LANSMART\DE-200.DRV  C:\LANSMART\LANSMA05.DRV

2. Now, you can use both the menu mode and the command mode to log onto

the network.  The LOGON command syntax is as follows:

> LOGON <username> <SRV|WKS> /NET:5 [other parameters]

2. RUN LANsmart version 2.xx & v3.xx

=====================================

You may use both the menu and the command mode to logon to the network.

The LOGON command syntax is as follows:

> LOGON <username> <SRV|WKS> /NET:DE-200 [other parameters]

The driver program DE-200.DRV should first be copied to the directory

that contains LOGON.EXE program. /NET:DE-200 specifies to LANsmart your

network adapter card.