July 1992

3Com 3+ Share Driver Installation

for

D-Link 16-Bit DE-200 series Ethernet Card

=========================================

The D-Link 3Com 3+ Share Driver Program lets you run 3Com 3+ Share

on your PC with the D-Link 16-Bit DE-200 series Ethernet card.

The following files will be used in this installation:

Files            Contents

-----------      -----------------------------

DE200PD.SYS      DE-200 Series Driver Program

ETH.SYS          3Com 3+ Share Driver

1. Follow the installation procedure for 3Com EtherLink board.

2. Copy the program ETH.SYS from the Installation Diskette for

the D-Link DE-200 series 16-Bit Ethernet card to the boot

disk/diskette.

This program will replace the program of the same file name

( ETH.SYS ) when you perform the installation procedure in

step 1 above.

3. In the CONFIG.SYS file on the boot disk/diskette, make sure

you have the following two statements.  The order of these

two statement must be as shown below.

DEVICE = DE200PD.SYS <packet_int_no> <int_level> <io_addr>

DEVICE = ETH.SYS

You may now boot and start your 3COM workstation.

Notes:

<packet_int_no> is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

<int_level>     is the hardware interrupt (IRQ) line set on your network

interface card. The default is 3 if this parameter is not

specified.

<io_addr>       is base I/O address set on your network interface card.

The default address of 0x300 is used, if this parameter

is not specified.