July 1992

NEXOS dirver Installation

for

D-Link DE-200 Series 16-Bit Ethernet card

=========================================

The following files will be used to in this installation:

Files           Contents

-----------     -----------------------------------------------------------

DE200PD.COM     Packet Driver program for DE-200 series card

LNXPD.EXE       NEXOS shell program

The D-Link driver programs above let you run NEXOS workstation

with the D-Link Ethernet 16-bit cards.  Two methods can be

used:

1. Execute the DE200PD.COM as follows:

>DE200PD <packet_int_no> <int_level> <io_addr>

2. Execute the LNXPD.EXE shell program.

> LNXPD

The detail operating procedure please refer to NEXOS manual.

Notes:

<packet_int_no> is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

<int_level>     is the hardware interrupt (IRQ) line set on your network

interface card. The default is 3 if this parameter is not

specified.

<io_addr>       is base I/O address set on your network interface card.

The default address of 300H is used, if this parameter

is not specified.