July 1992

FTP PC/TCP Installation

for

D-Link 16-Bit DE-200 series Ethernet Card

=========================================

The following files will be used in this installation:

Files         Contents

-----------   ---------------------------------

DE200PD.SYS   DE-200 series Driver Program

DE200PD.COM   DE-200 series Driver Program

The D-Link packet driver programs above let you run FTP's PC/TCP

with the D-Link DE-200 series Ethernet Card.  Two methods can be used:

METHOD 1:

If you use the DE200PD.SYS program, the following statement must

be included in your CONFIG.SYS file:

DEVICE = DE200PD.SYS <packet_int_no> <int_level> <io_addr>

This statement may be placed anywhere in the CONFIG.SYS file.

METHOD 2:

If you use the DE200PD.COM program, follow the procedure below:

1. Execute the DE200PD.COM as follows:

>DE200PD <-d> <packet_int_no> <int_level> <io_addr>

2. Execute the ETHDRV.EXE device driver program (provided by

FTP).

You are now ready to use FTP's PC/TCP.

Notes:

<packet_int_no> is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

<int_level>     is the hardware interrupt (IRQ) line set on your network

interface card. The default is 3 if this parameter is not

specified.

<io_addr>       is base I/O address set on your network interface card.

The default address of 0x300 is used, if this parameter

is not specified.

<-d>            Delay Initial - The driver program will not initialize

the network interface card until the packet driver program

is accessed by its upper layer program first time.