NetWare Driver Installation                 July 1992

===========================

The NetWare driver programs in this disk support the following D-Link cards:

DE-200

DE-200TP

DE-200+

DE-200TP+

DE-205TP

This diskette is labeled as LAN_DRV_DLK and the files are organized as follows:

Directory             Contents

---------             --------------------------------------------------------

\NETWARE              A pre-generated IPXDE200.COM

\NETWARE\V2xx         NetWare v2.15b, v2.15c or v2.2 server & bridge drivers

\NETWARE\V215b        NetWare v2.15b workstation drivers

\NETWARE\V215C        NetWare v2.15c, v2.2 or NetWare 386 workstation drivers

\NETWARE\NET386       NetWare 386 server or bridge drivers

\NETWARE\ODI.DOS      ODI workstation driver for DOS

\NETWARE\ODI.OS2      ODI workstation driver for OS/2

\NETWARE\V20A\SRV

\NETWARE\V20A\WKS

*********************************

*  HOW TO SETUP IPX             *

*********************************

The IPXDE200.COM file can be used on Novell NetWare Lite,

NetWare 2.1x, NetWare 2.2 and NetWare 386 3.xx workstation configuration.

And you can run the SET200.EXE program to create an IPX.COM file

that matches the DE-200 configuration to replace the IPXDE200.COM .

METHOD:

Insert the DE-200 driver programs diskette in your floppy drive and change

the directory to \NETWARE to run the SET200.EXE program.

> SET200

-- Upon choosing the hardware configuration, IPX.COM file will

be created in your current directory.  Use this IPX.COM

file to access the Novell network.

*********************************

*  QUICK SETUP FOR WORKSTATION  *

*********************************

A pre-generated workstation driver IPXDE200.COM is included in this disk.

You can simply run the IPXDE200 on a workstation to access the Novell network:

>IPXDE200    --Supports NetWare Lite, 2.1x, 2.2 or NetWare 386

>NETx        --This program is provided by Novell

***********************

*  WORKSTATION SETUP  *

***********************

If you want to generate your own workstation driver instead of using the

provided IPXDE200.COM, please do the following:

1)  If you make a backup of this diskette, make sure that it is labeled as

LAN_DRV_DLK.  You can use the DOS "Label" command as follows:

>Label  A:LAN_DRV_DLK

2)  Copy the appropriate NetWare driver files to the root directory of

this diskette (or backup):

>Copy  A:\NETWARE\V215c\*.*  A:\   --For NetWare 386 workstation

3)  Execute Novell's SHGEN or WSGEN program, which resides in the SHGEN-1 or

WSGEN diskette.  When you are asked to select a network driver, press

the INS key.  You will be asked to insert a LAN_DRV_??? diskette.  Insert

this diskette (or backup) in drive A.  Upon completion, a new IPX.COM file

will be created in your SHGEN-2 or WSGEN diskette.

Note : if you generate IPX.COM by "1:Driver Configurable by JUMPERS Utility"

the I/O PORT # will be None, then you must execute JUMPERS Utility

offered by NETWARE, the procedure as follow:

a. >JUMPERS

b. Enter the file "IPX.COM" to modify

c. Screen display as follow:

DE-200 Options

I/O PORT #1     None

d. press Enter

(window assistance)

I/O PORT#

=========

200

220

240

260

280

300

.

.

.

e. Select one port#

f. Save the port# that you selected.

************************

*  NETWARE LITE USERS  *

************************

1)  Follow the normal installation procedures as instructed by NetWare Lite.

When asked to select a network driver, simply choose "NOVELL NE2000".

2)  After installation completed, modify the STARTNET.BAT file as follows:

From:

---------------------------

LSL

NE2000

IPXODI A

CLIENT

To:

---------------------------

LSL

DE200ODI

IPXODI A

CLIENT

3) Copy \NETWARE\ODI.DOS\NET.CFG file to NWLITE sub-directory which was

created when you installed NetWare Lite.

4) You may run STARTNET to access the Novell network.

*****************************

*  SERVER AND BRIDGE SETUP  *

*****************************

Example 1:  NetWare 386 Server/Bridge Setup

===========================================

1)  Copy the appropriate NetWare driver files (from this diskette) to the

root directory of your Novell diskette labeled SYSTEM-1.

>Copy  A:\NETWARE\NET386\*.*  B:\   --SYSTEM-1 disk must be in drive B

2)  Execute Novell's SERVER program, which resides in the SYSTEM-1 diskette.

After loading the hard disk driver, you can load the network driver as

follows:

:Load  A:\386DE200.LAN     --Select your network card settings

:Bind ipx to 386DE200

:load install              --Follow the NetWare installation instructions

Example 2:  NetWare v2.2 Server/Bridge Setup

============================================

1)  Copy the appropriate NetWare driver files (from this diskette) to the

root directory of the diskettes labeled SYSTEM-1 and LAN_DRV_DLK.

>Copy A:\NETWARE\V2XX\*.LAN  B:\    --SYSTEM-1 disk must be in drv B

>Copy A:\NETWARE\V2XX\*.OBJ  A:\    --Copy the files to the root dir

2)  Execute Novell's INSTALL program, which resides in the SYSTEM-1 diskette.

Choose the "D-Link Ethernet DE-200" driver when asked for a network driver.

Example 3:  NetWare v2.15b Server/Brige Setup

=============================================

1)  Copy the appropriate NetWare driver files (from this diskette) to the

root directory of your Novell diskette labeled LAN_DRV_DLK.

>Copy  A:\NETWARE\V2XX\*.LAN  B:\ --AUXGEN disk must be in drive B

>Copy  A:\NETWARE\V2XX\*.OBJ  A:\ --LAN_DRV_DLK disk must be in drive A

2)  Execute Novell's NETGEN program, which resides in the NETGEN diskette.

Choose the "D-Link Ethernet DE-200" driver, when asked for a network driver.

************************************

* Install NetWare V2.2 workstation *

************************************

Refer to procedure  A. HOW TO SETUP IPX and follow all procedures

except for #3.  Instead of running the program SHGEN.EXE, run WSGEN.EXE .

**********************

*  ODI DRIVER SETUP  *

**********************

ODI Workstation for DOS

=======================

D-Link ODI Driver program allows you to run multiple protocl stacks.  It also

allows you to install up to two network adapters in a workstation. D-Link ODI

driver supports "port, node address, protocol and frame" options.  You can use

these options to create your own NET.CFG file.  For detailed info on Keywords,

please refer to Novell manual "NetWare ODI shell for DOS - NET.CFG options".

1)  Copy the DE200ODI.COM file (from this diskette) to the directory which

contains Novell's LSL.COM, IPXODI.COM and NETx.COM files.

2)  Follow Novell's instructions on using the ODI programs. You can substitute

the DE200ODI.COM whenever a reference is made to the Hardware Specific Module.

3)  A typical startup for ODI workstation is as follows:

>LSL               --Link Support Layer Module provided by Novell

>DE200ODI          --Hardware Specific Module provided by D-Link

>IPXODI            --IPX Protocol Stack Module provided by Novell

>NETx              --DOS Support Module provided by Novell

>LOGIN

ODI Workstation for OS/2

========================

1)  You must first install the Requester on the workstation.  For detailed

information, refer to Novell manual "NetWare Requester for OS/2".

2)  Copy DE200ODI.SYS (from this diskette) to your C:\NETWARE directory.

Afterwards, add the following device driver statement to your CONFIG.SYS

file:

DEVICE=C:\NETWARE\DE200ODI.SYS

3)  D-Link ODI driver supports "port, node address, protocol and frame"

options.  You can use these options to create your own NET.CFG file. For

detailed information on Keywords, please refer to Novell manual

"NetWare Requester for OS/2 - Reconfiguring the NetWare Requester".