February 1995

FTP PC/TCP Installation

for

DE-530 Ethernet Adapter

===============================

The following files will be used in this installation:

Files            Contents

------------    ---------------------------------

DE530PD.SYS      Packet Driver Program

DF530PD.COM      Packet Driver Program

The D-Link packet driver programs above let you run FTP's PC/TCP

with the D-Link DE-530 Ethernet Adapter.

Two methods can be used:

METHOD 1:

If you use the DE530PD.SYS program, the following statement must

be included in your CONFIG.SYS file:

DEVICE = DE530PD.SYS <-d> <-w> <-int:##>

This line may be placed anywhere in the CONFIG.SYS file.

METHOD 2:

If you use the DF530PD.COM program, follow the procedure below:

1. Execute the DE530PD.COM as follows:

>DE530PD <-d> <-w> <-int:##>

2. Execute the ETHDRV.EXE device driver program (provided by FTP Software).

You are now ready to use FTP's PC/TCP.

<-int:##>       is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

Note: '##' can be hex or decimal number.

hex - start with '0x'.            ex: -int:0x62

dec - start with nonzero digit.   ex: -int:98

<-slot:##>      is the number of assigned slot,which is from 16 to 26.

(only supports PCI bus).

Note: '##' can be hex or decimal number.

hex - start with '0x'.            ex: -slot:0x10

dec - start with nonzero digit.   ex: -slot:16

<-d>            Delay Initial - The driver program will not initialize

the network interface card until the packet driver program

is accessed by its upper layer program first time.

<-w>            the -w parameter is for use with the Windows environment only.

<-u>            is to unload the driver program from memory. The program will

automatically search for Packet driver to unload.

To select media type, you can use :

<-autodetect>   The driver program will automatically detect between AUI, TP

and BNC according to the active line.

Note: If you have no parameter about media selection,

the dirver will perform autodetection as default.

<-tp>           Force the driver to select Twisted Pair interface.

<-bnc>          Force the driver to select BNC interface.

<-aui>          Force the driver to select AUI interface.

<-tp_full_duplex>

Selects Twisted Pair interface running in FullDuplex mode.

<-tp_no_link_test>

Selects Twisted Pair interface. Transmit does not perform

link pulses test.