NDIS 2 MAC DRIVER for DE-530/DFE-500  V2.00

****************************************************

Note that this installation kit includes both the DOS

and OS/2 NDIS2 driver and this document describes

installation under both environments.  This revision

supersedes the DOS and OS/2 NDIS2 driver version 1.03 .

NOTE: The generic term "DC21X4" relates to the DECchip

21040, DECchip 21041 and DECchip 21140 Ethernet and

Fast Ethernet LAN controllers.

.  Installing the NDIS2 OS/2 driver.

.  Installation with Microsoft LAN Manager 2.x for OS/2

.  Automatic installation using the LAN Manager Setup Program:

1. In the \LANMAN directory, start the SETUP program

2. Select <Configuration> from the Main Menu.

3. Select <Network Driver> from the Configuration Menu.

4. Remove the old adapter using <Remove Configuration>.

5. Select <Add Configuration>.

6. Select <Other Drivers> and follow the directions.

. Manual Installation for OS/2 LAN Manager

If LAN Manager is already installed and configured and you do not

wish to use the setup program, perform the following manual steps:

1. Create DC21X4 Driver directory

<DRIVE>:\LANMAN\DRIVERS\ETHERNET\DC21X4.

2. Copy the files

\MSLANMAN.OS\DRIVERS\ETHERNET\DC21X4\DC21X4.OS2

and

\MSLANMAN.OS\DRIVERS\ETHERNET\DC21X4\PROTOCOL.INI

to the new directory.

3. Copy the file

\MSLANMAN.OS\DRIVERS\NIF\DC21X4.NIF

to the directory

<DRIVE>:\LANMAN\DRIVERS\NIF.

4. Add the following line to your CONFIG.SYS to load the

driver:

DEVICE=<DRIVE>:\LANMAN\DRIVERS\ETHERNET\DC21X4\DC21X4.OS2.

5. Modify your PROTOCOL.INI file using any OS/2 Editor by

appending the PROTOCOL.INI stub (included with this kit) to

the end of your old PROTOCOL.INI file.  The stub contains

the following information:

[DC21X4_NIF]

DRIVERNAME=DC21X4$

6. Modify TCPIP_XIF and NETBEUI_XIF modules in the PROTOCOL.INI

file by changing the value of the BINDINGS keyword

from  BINDINGS = driverlabel

to    BINDINGS = DC21X4_NIF