Using the DE-530CT Adapter 
with LANsmart 3.x

In order to use your DE-530 NIC you need to use an [Netware] ODI stack.  Download “ls3x530.exe” from our BBS or our Web Site.

BBS:	714.455.1779  /9600 baud, 8N1

Web:	http://www.dlink.com

- Install LANsmart as you normally would. When coming to a point of choosing your NIC (Network Interface Card), pick any Ethernet adapter. This selection won’t affect your network but only serves to allow the install process to continue.

- Allow LANsmart to create its directory, install all of its files and configure your node.

- Execute ls3x530.exe which will auto-extract all the necessary files.

- Then execute install.bat from your c:\ root directory. This will place all the necessary files in your c:\lansmart directory. Finally any residual extracted files will be removed.

- Next step, your c:\startup.bat file should look like this...

- cd \lansmart

lsl.com

dlkpci.com

ipxodi.com

logon <username>	[your username]

netshare1 resource1=c:\<path>	[if running a Server]

connect2 d: \\username\resource1\<path> /b	[if running a Workstation]

1)	Netshare configures a local resource i.e. a hard drive or printer to be shared over the network.  Resource1 can be any name you wish to assign to the drive or directory to be shared — e.g. server2, marketing, or r&d or whatever you choose.

The path is only necessary if you want to share a specific directory e.g.  c:\dos or c:\windows\system, etc. as opposed to sharing the entire c: drive.

Sharing of resources is only possible if your station is configured as either a Dedicated Server or as a File/Print Server.

2)	Connect attaches a Workstation to resources shared on the network.  Assuming that you have only one local drive c: (no C-ROM drive, no backup drive), the first drive letter that can be mapped to a network resource would be d:.

The connect syntax consists of the File Server’s username (the name this person uses in their logon command) and the name of the resource they are sharing.

The “/b” switch allows you to connect to a resource as soon as it becomes available (in case you log on before the resource is shared).

Should any questions remain, you may direct them to D-Link’s Technical Support Team at the above locations.