Jul. 1995

D-Link Driver Installation Readme

for Windows '95

=============================================================================

NOTE 1: This driver kit will only work on Beta 2 or later.

NOTE 2: When install network card driver, Win95 CD ROM or floppies should

exists.

=============================================================================

0. Copy NETD62E.INF from A:\WIN95 to C:\WINDOWS\INF if the Windows path is

in C:\WINDOWS.

1. From the Control Panel, select Network. The Network windows should appear.

2. From the Network window, click on "Add". You will then be asked to select

a component type, double click on  "Adapter".

3. A list of network card manufacturers and model numbers should now be shown.

Insert the driver disk in A: and click on "Have Disk". At the

install from disk menu, enter A:\WIN95 for the location of manufacturers

information. Click on "OK". A list of D-Link network cards should appear.

4. Select the appropriate card for your system and click "OK".

5. While copying D-Link drivers, Win95 also need Win95 CD ROM to copy other

files in  D:\Retail if your CD ROM drive is D:. So you need to change the

corresponding path(A:\WIN95 or D:\RETAIL).

6. When all the setting are complete, click on "OK" on the properties page

to return to the network window. If everything looks correct here, click

on "OK" to close the network window.