Jul. 1995

3Com 3+ Share Driver Installation/

FTP PC/TCP Installation/

PC-NFS Driver Installation

for

D-Link DE-621 Duplex and DE-620

==================================

Thank you for purchasing the D-Link Duplex for desktop, laptop

and notebook PCs to retain DE-620 and printer or key card connection.

Before you installing driver program, please refer to DE-620 or DE-621

manual to get more detail installation. A simple installation is as the

following figure.

����

/	/�

/    /  �

/    /	*�<------- connect to DE-620 only

����Ŀ  *  �

connect     |\ 	      �    �*	*�<------- connect to printer or

to pc	     |.|	  �����    �  *  /	   key card

printer <-- |.|���������ͳ DE-621 �*  /

port	     |.|	  �	   � /

|/ 	  ����������

Note: DE-620 series product cannot Ping to server when it works with

IBM PS/VP.

***********************************************

How to install DE-620 with 3Com 3+ Share driver

***********************************************

The D-Link 3COM 3+ Share Driver Program lets you run 3Com 3+ Share

on your PC with the D-Link Ethernet Pocket LAN Adapter.

The following files will be used in this installation:

Files		 Contents

-----------	 -------------------------

DE620PD.SYS	 DE-620 Driver Program

ETH.SYS 	 3Com 3+ Share Driver

1. Follow the installation procedure for 3Com EtherLink board.

2. Copy the program ETH.SYS from the Installation Diskette for

the D-Link Ethernet Pocket LAN Adapter to the boot disk/diskette.

This program will replace the program of the same file name

(ETH.SYS) when you perform the installation procedure in step 1

above.

3. In the CONFIG.SYS file on the boot disk/diskette, make sure

you have the following two statements.  The order of these two

statement must be as shown below.

DEVICE = DE620PD.SYS <packet_int_no>

DEVICE = ETH.SYS

You may now boot and start your 3COM workstation.

Notes:

<packet_int_no> is the software interrupt vector number used by the

packet driver program, which can be from 96 to 128.

*************************************

How to install DE-620 with FTP PC/NFS

*************************************

The following files will be used in this installation:

Files		Contents

-----------	----------------------------------------------------------

PDNFS.SYS	PC-NFS Driver Program

DE620PD.SYS	Packet driver program for DE-620 card

DE620PD.COM	Packet driver program for DE-620 card

Two methods can be used to install:

METHOD 1:

1. Install the PC-NFS program according to the procedure described in the

PC-NFS manual.  On the screen of the PC-NFS install program, select 3C501

(the 3Com EtherLink card) as your network interface card. When the screens

prompt you, select 3 for the interrupt level (IRQ), and select 300h for

the base I/O address.

2. Copy the following files to the \NFS directory of your hard

disk:

PDNFS.SYS

DE620PD.SYS

DE620PD.COM

3. Modify the CONFIG.SYS file as follows:

DEVICE=C:\ANSI.SYS

DEVICE=C:\NFS\PCNFS.SYS /m

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE=C:\NFS\VECIE6.SYS

DEVICE=C:\NFS\DE620PD.SYS <packet_int_no>	<<---- DE-620 packet driver

DEVICE=C:\NFS\NFSVEC.SYS

DEVICE=C:\NFS\PDNFS.SYS			<<---- PCNFS driver program

LASTDRIVE=V

AUTOEXEC.BAT file:

SET PATH = C:\; C:\NFS

SET TZ=EST5EDT

SET NFSDRIVE = C

NET YPDOMAIN ypserver

NET START RDR

NET YPSET homer

NET NAME john

NET USER F: \\user\john

We use the name "john" here as an example.

Notes:

<packet_int_no> is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

4. Reboot the PC to start PC-NFS.

You may now execute the PC-NFS commands to communicate

other computers through the TCP/IP protocol.

METHOD 2:

1. Install the PC-NFS program according to the procedure described in the

PC-NFS manual.  On the screen of the PC-NFS install program, select 3C501

(the 3Com EtherLink card) as your network interface card. When the screens

prompt you, select 3 for the interrupt level (IRQ), and select 300h for

the base I/O address.

2. Copy the following files to the \NFS directory of your hard

disk:

PDNFS.SYS

DE620PD.SYS

DE620PD.COM

3. Modify the CONFIG.SYS file as follows:

DEVICE=C:\ANSI.SYS

DEVICE=C:\NFS\PCNFS.SYS /m

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE=C:\NFS\VECIE6.SYS

DEVICE=C:\NFS\NFSVEC.SYS

DEVICE=C:\NFS\PDNFS.SYS 	 <<----- PCNFS driver program

LASTDRIVE=V

AUTOEXEC.BAT file:

SET PATH = C:\; C:\NFS

SET TZ=EST5EDT

SET NFSDRIVE = C

DE620PD <packet_int_no> 	 <<----- DE-620 packet driver

NET YPDOMAIN ypserver

NET START RDR

NET YPSET homer

NET NAME john

NET USER F: \\user\john

We use the name "john" here as an example.

Notes:

<packet_int_no> is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

4. Reboot the PC to start PC-NFS.

You may now execute the PC-NFS commands to communicate

other computers through the TCP/IP protocol.

*************************************

How to install DE-620 with FTP PC/TCP

*************************************

Thank you for purchasing the D-Link Duplex for desktop, laptop

and notebook PCs to retain DE-620 and printer or key card connection.

Before you installing driver program, please refer to DE-620 or DE-621

manual to get more detail installation. A simple installation is as the

following figure.

����

/	/�

/    /  �

/    /	*�<------- connect to DE-620 only

����Ŀ  *  �

connect     |\ 	      �    �*	*�<------- connect to printer or

to pc	     |.|	  �����    �  *  /	   key card

printer <-- |.|���������ͳ DE-621 �*  /

port	     |.|	  �	   � /

|/ 	  ����������

The following files will be used in this installation:

Files	      Contents

-----------   ---------------------------------

DE620PD.SYS   DE-620 series Driver Program

DE620PD.COM   DE-620 series Driver Program

The D-Link packet driver programs above let you run FTP's PC/TCP

with the D-Link DE-620 series Ethernet Card.  Two methods can be used:

METHOD 1:

If you use the DE620PD.SYS program, the following statement must

be included in your CONFIG.SYS file:

DEVICE = DE620PD.SYS <packet_int_no>

This statement may be placed anywhere in the CONFIG.SYS file.

METHOD 2:

If you use the DE620PD.COM program, follow the procedure below:

1. Execute the DE620PD.COM as follows:

>DE620PD <packet_int_no>

2. Execute the ETHDRV.EXE and IDRIVE.EXE device driver programs (provided by

FTP).

You are now ready to use FTP's PC/TCP.

Notes:

<packet_int_no> is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.