NetWare Driver Installation 		Jul. 1995

for

DE-621 Duplex and DE-620 Ethernet Pocket LAN Adapter

====================================================

Thank you for purchasing the D-Link Duplex for desktop, laptop

and notebook PCs to retain DE-620 and printer or key card connection.

Before you installing driver program, please refer to DE-620 or DE-621

manual to get more detail installation. A simple installation is as the

following figure.

����

/	/�

/    /  �

/    /	*�<------- connect to DE-620 only

����Ŀ  *  �

connect     |\ 	      �    �*	*�<------- connect to printer or

to pc	     |.|	  �����    �  *  /	   key card

printer <-- |.|���������ͳ DE-621 �*  /

port	     |.|	  �	   � /

|/ 	  ����������

Note: The function of DE-621 doesn't include in the Netware 386 server,

bridge driver and OS/2 version driver.

The NetWare driver programs in this disk support the following D-Link cards:

DE-620,DE-620CT,DE-620TP,DE-620AUI,

This diskette is labeled as LAN_DRV_DLK and the files are organized as follows:

Directory		      Contents

-----------------------       ------------------------------------------------

\NETWARE\CLIENT 	      A pre-generated IPXDE620.COM

\NETWARE\SERVER\NW311	      NetWare 386 server or bridge drivers

\NETWARE\CLIENT\V215B	      NetWare v2.15b workstation drivers

\NETWARE\CLIENT\V215C	      NetWare v2.15c, v2.2 or NetWare 386 workstation drivers

\NETWARE\CLIENT\ODI.DOS       ODI workstation driver for DOS (support Netware

v3.11, v3.12 and v4.x)

\NETWARE\CLIENT\ODI.OS2       ODI workstation driver for OS/2

*********************************

*  QUICK SETUP FOR WORKSTATION	*

*********************************

A pre-generated workstation driver IPXDE620.COM is included in this disk.

You can simply run the IPXDE620 on a workstation to access the Novell network:

>IPXDE620	  --Supports NetWare Lite, 2.1x, 2.2 or NetWare 386

>NETx	  --This program is provided by Novell

***********************

*  WORKSTATION SETUP  *

***********************

If you want to generate your own workstation driver instead of using the

provided IPXDE620.COM, please do the following:

1)  If you make a backup of this diskette, make sure that it is labeled as

LAN_DRV_DLK.  You can use the DOS "Label" command as follows:

>Label  A:LAN_DRV_DLK

2)  Copy the appropriate NetWare driver files to the root directory of

this diskette (or backup):

>Copy  A:\NETWARE\CLIENT\V215C\*.*  A:\	--For NetWare 386 workstation

3)  Execute Novell's SHGEN or WSGEN program, which resides in the SHGEN-1 or

WSGEN diskette.  When you are asked to select a network driver, press

the INS key.  You will be asked to insert a LAN_DRV_??? diskette.  Insert

this diskette (or backup) in drive A.  Upon completion, a new IPX.COM file

will be created in your SHGEN-2 or WSGEN diskette.

************************

*  NETWARE LITE USERS  *

************************

1)  Follow the normal installation procedures as instructed by NetWare Lite.

When asked to select a network driver, simply choose "NOVELL NE2000".

2)  After installation completed, modify the STARTNET.BAT file as follows:

From:

---------------------------

LSL

NE2000

IPXODI A

CLIENT

To:

---------------------------

LSL

DE620ODI

IPXODI A

CLIENT

3) Copy \NETWARE\CLIENT\ODI.DOS\NET.CFG file to NWLITE sub-directory which was

created when you installed NetWare Lite.

4) You may run STARTNET to access the Novell network.

************************************

* Install NetWare V2.2 workstation *

************************************

Refer to procedure of Workstation Setup: HOW TO SETUP IPX and follow all procedures

except for #3.	Instead of running the program SHGEN.EXE, run WSGEN.EXE .

**********************

*  ODI DRIVER SETUP  *

**********************

ODI Workstation for DOS

=======================

D-Link ODI Driver program allows you to run multiple protocol stacks.  It also

allows you to install up to two network adapters in a workstation. D-Link ODI

driver supports "port, node address, protocol and frame" options.  You can use

these options to create your own NET.CFG file.	For detailed info on Keywords,

please refer to Novell manual "NetWare ODI shell for DOS - NET.CFG options".

1)  Copy the DE620ODI.COM file (from this diskette) to the directory which

contains Novell's LSL.COM, IPXODI.COM and NETx.COM files.

2)  Follow Novell's instructions on using the ODI programs. You can substitute

the DE620ODI.COM whenever a reference is made to the Hardware Specific Module.

3)  A typical startup for ODI workstation is as follows:

>LSL		--Link Support Layer Module provided by Novell

>DE620ODI		--Hardware Specific Module provided by D-Link

>IPXODI		--IPX Protocol Stack Module provided by Novell

>NETx		--DOS Support Module provided by Novell

>LOGIN

ODI Workstation for OS/2

========================

1)  You must first install the Requester on the workstation.  For detailed

information, refer to Novell manual "NetWare Requester for OS/2".

2)  Copy DE620ODI.SYS (from this diskette) to your C:\NETWARE

directory. Afterwards, add the following device driver statement to your

CONFIG.SYS file:

DEVICE=C:\NETWARE\DE620ODI.SYS

3)  D-Link ODI driver supports "port, node address, protocol and frame"

options.  You can use these options to create your own NET.CFG file. For

detailed information on Keywords, please refer to Novell manual

"NetWare Requester for OS/2 - Reconfiguring the NetWare Requester".

*****************************

*  SERVER AND BRIDGE SETUP  *

*****************************

Example :  NetWare 386 Server/Bridge Setup

===========================================

1)  Copy the appropriate NetWare driver files (from this diskette) to the

root directory of your Novell diskette labeled SYSTEM-1.

>Copy  A:\NETWARE\SERVER\NW311\*.*  B:\	--SYSTEM-1 disk must be in drive B

2)  Execute Novell's SERVER program, which resides in the SYSTEM-1 diskette.

After loading the hard disk driver, you can load the network driver as

follows:

:Load  A:\386DE620.LAN	--Select your network card settings

:Bind ipx to 386DE620

:load install		--Follow the NetWare installation instructions

******************************

* NetWare ODI CLIENT DRIVERS *

******************************

ODI Client for DOS

===================

D-Link ODI Driver allows you to load multiple protocol stacks. It also allows

you to install up to two network adapters in a workstation. D-Link ODI driver

supports "port, node address and frame" options. You can use these options to

customize the NET.CFG file.  For detailed info on Keywords, please refer to

Novell manual "NetWare ODI shell for DOS - NET.CFG options".

Please install the NetWare ODI client driver files following the procedures

as described in the NetWare installation guide.

If you want to generate your own NetWare ODI client driver, please do the

following:

1)  Make a backup of WSDRV_2 diskette.

2)  Copy the appropriate DE620ODI.COM, DLKDRVS.DOS and DE620ODI.INS driver

files to DOS directory of WSDRV_2 diskette (or backup):

Insert D-Link driver diskette in A: driver.

Insert NetWare WSDRV_2 diskette in B: driver.

>Copy  A:\NETWARE\CLIENT\ODI.DOS\*.*    B:\DOS  --For NetWare client workstation

Append contents of DLKDRVS.DOS to DRIVERS.DOS

Contents of DLKDRVS.DOS is as follows:

" D-Link DE-100 Ethernet for ISA,DE100ODI.COM,DE100ODI.INS,IE0

D-Link DE-200 Ethernet for ISA,DE200ODI.COM,DE200ODI.INS,IE0

D-Link DE-220 Series Ethernet for ISA,DE220ODI.COM,DE220ODI.INS,IE0

D-Link DE-320 Ethernet for Micro Channel,DE320ODI.COM,DE320ODI.INS,00M

D-Link DE-400 Ethernet for EISA,DE400ODI.COM,DE400ODI.INS,0E0

D-Link DE-600 Ethernet Pocket LAN,DE600ODI.COM,DE600ODI.INS,IEM

D-Link DE-620 Ethernet Pocket LAN,DE620ODI.COM,DE620ODI.INS,IEM

D-Link DE-650 Ethernet PCMCIA,DE65NODI.COM,DE65NODI.INS,IEM 	    "

3)  Execute Novell's INSTALL program, which resides in the WSDRV_2 diskette.

When you are asked to install the driver for your network board, press

the INS key.  You will be asked to insert a WSDRV_2 diskette.  Insert

this diskette (or backup) which contains DE620ODI.COM, DRIVERS.DOS and

DE620ODI.INS in drive A.

4)  Follow the NetWare Client Install normal installation procedures, Which

resides in the WSDOS_1 diskette. When asked to select a network driver,

simply choose "D-Link DE-620 Pocket Ethernet Card" for selecting

parameter like as Base I/O Port, Media Frame Type(s) and Optional Node

Address.

Upon completion, a new NET.CFG file will be created in your DOS directory.

A typical of NET.CFG is as follows:

Link Driver DE620ODI

FRAME Ethernet_802.2

NetWare DOS Requester

FIRST NETWORK DRIVE = F

:

:

5)  A typical startnet.bat for ODI workstation is as follows:

SET NWLANGUAGE=ENGLISH  --Set NetWare 4.0 to English language

LSL		     --Link Support Layer Module provided by Novell

DE620ODI		     --Hardware Specific Module provided by D-Link

IPXODI		     --IPX Protocol Stack Module provided by Novell

VLM		     --DOS Support Module provided by Novell

F: 		     --Change to connected device

LOGIN .ADMIN.NV40SRV    --Login to file server NV40SRV as user ADMIN

6) You may run STARTNET to access the Novell network.