D-Link Windows 95 install kit                                     Sept. 1995

D-Series Adapter Install for Windows 95

���������������������������������������

Contents

# Introduction

# Check your Compatibility

# Prepare for the Install

# Remove any previous network setup

# Check your PCMCIA support (For PCMCIA users only)

# Add D-Link Adapter Support

# Troubleshooting Tips

# Reported Bugs

# Appendix for Support Access

Introduction

Below is a very detail install method for D-Link Adapters.

There are 5 steps involved in making your install seamless

as possible. DO NOT skip any step or it will lead to more

time and effort spent on the install instead of using our

adapters. Use the following method to install, if your

adapter doesn't work try it once more step by step. If your

install is not successful on the second try, call D-Link

immediately to recieve support. Very few mistakes can be

made with Windows '95 and support can probably have

it working in a very short time.

Compatibility

# Compatible Adapters

DE-100 Series            DE-400

DE-150                   DE-500/510

DE-200 Series            DE-520/530

DE-220 Family Series     DFE-500TX

DE-250                   DE-600/620

DE-300/320               DE-650CT (dual media only)

# Future Support (by 10/31/95)

DE-650C/DE-650T     DME-650T

DT-650U/DT-650S     DT 100/220/220+

# Incompatibile

DA Series Adapters  DX Series Adapters

DN Series Adapters

Prepare for Installtion

# Install Windows 95 in the target computer.

# From the BBS download DLKWIN95.EXE

D-Link's BBS 714-455-1779 (9600-N-8-1)

714-455-9616 (9600-N-8-1)

# Run DLKWIN95.EXE will self-extract to a \DLKWIN95

directory off the root of the drive it is executed.

# Copy the NETDLINK.INF in \DLKWIN95 to the \INF in your

Windows 95 directory. Use Explorer find \INF if needed.

Remove Any Previous Network Support

# Power off the target computer.

# Remove any network adapter card from the machine.

# Power on Windows 95 to the standard 95 desktop screen.

# Click "Start" on the main tool bar.

# Click "Settings" on the main selection list.

# Click "Control Panel" on the "Settings" selection list.

# Under the "Control Panel" group, double click "NETWORK" icon.

- Skip to next step if the "NETWORK" icon is missing.

- Highlight the network adapter and Press the remove button.

- This should leave a blank network support window.

- Press OK to exit

- Say Cancel to reboot.

# Under Control Panel double click on the "SYSTEM" icon.

# Select the "Device Manager" tab at the top.

# Inspect the list for Network Adapter(s)

# If the list has a Network Adapter expand the list

by pressing the + next to "Network Adapter"

# Highlight any adapters and press Remove.

# Continue this until all network adapters are removed.

# Press "Close" when completed.

# Press "Start" on the main tool bar and Shutdown

on main selection list.

# Choose "Restart" and the system should reboot.

Check your PCMCIA Support (For NON-PCMCIA users skip to next section)

# Power off the target computer.

# Remove any network adapter card from the machine.

# Power on Windows 95 to the standard 95 desktop screen.

# Click "Start" on the main tool bar.

# Click "Settings" on the main selection list.

# Click "Control Panel" on the "Settings" selection list.

-- Control Panel will open up a window of many icons.

# Under Control Panel double click on the "SYSTEM" icon.

# Select the "Device Manager" tab at the top.

# Inspect the list for PCMCIA socket. If this is missing

you will have install it with aid of your PCMCIA

hardware manufacture.

# If the list has a PCMCIA socket expand the list by

pressing the + next to "PCMCIA socket"

# Typically there will only be one PCMCIA driver, highlight

the driver and select properties.

# In the "Device Status" it must say the it is working

properly. If it is not contact the PCMCIA's manufacture.

D-Link will not be able to help you until this is

working properly.

# If the socket is running properly, cancel out to the

control panel group.

# Press "Start" on the main tool bar and Shutdown on main

selection list.

# Choose "Shutdown"

Adding D-Link Adapter Support

# Install the adapter into your machine.

-Set up jumpered adapters before installation.

-Adapters requiring software setup, need to be setup

in DOS based machines. If you upgraded from DOS

press F8 during boot up for 95's ability to boot MS DOS.

Setup the adapter to unique settings and continue.

-If you have a PCMCIA adapter simply insert the adapter.

# Boot computer and Windows 95 will come up.

# One of the following will happen:

1# Windows 95 will automatically recognize the card

install the drivers, and reboot.

2# Windows 95 will automatically recognize the card

but require you to select a driver.

3# Windows 95 doesn't recognized any new hardware.

# If 1# occurs congradulations you have installed the

adapter and it should be functioning. See below for

troubleshooting.

# If 2# occurs select "disk provided by hardware manufacture"

-Input the location of the drivers ie. C:\DLKWIN95

-Select the appropriate card

-It may ask for a number of files from the Windows 95 disks

or from the \WIN95 directory on the CD-Rom release.

-After it is finished it should ask to reboot. If it doesn't

ask to reboot, choose "Start", "Shutdown", and restart

the computer.

-It should be functioning. See below for Troubleshooting.

# If 3# occurs:

- Click "Start" on the main tool bar.

- Click "Settings" on the main selection list.

- Click "Control Panel" on the "Settings" selection list.

- Under the "Control Panel" group, double click

the "Add New Hardware" icon.

- Click "Next"

- Click "No" for the search new hardware

- Click "Next"

- On the list for Hardware Types click "Network Adapters"

- On the list for Manufactures click "D-Link Corporation"

- On the list for Models click your adapter model

-Input the location of the drivers ie. C:\DLKWIN95

-Select the appropriate card

-It may ask for a number of files from the Windows 95 disks

or from the \WIN95 directory on the CD-Rom release.

-After it is finished it should ask to reboot. If it doesn't

ask to reboot, choose "Start", "Shutdown", and restart

the computer.

-It should be functioning. See below for Troubleshooting.

Troubleshooting Tips

# Power off the target computer.

# Remove any network adapter card from the machine.

# Power on Windows 95 to the standard 95 desktop screen.

# Click "Start" on the main tool bar.

# Click "Settings" on the main selection list.

# Click "Control Panel" on the "Settings" selection list.

-- Control Panel will open up a window of many icons.

# Under Control Panel double click on the "SYSTEM" icon.

# Select the "Device Manager" tab at the top.

# Inspect the list for Network Adapter. If this is missing

you will have not installed Network adapter support,

please see above to add support.

# If the list has Network Adapters expand the list by

pressing the + next to "Network Adapters"

# Typically there will only be one Network driver, highlight

the driver and select properties.

# In the "Device Status" it must say the it is working

properly. Follow the advice of troubleshooting if it

is not working. Windows will complain if you select

conflicting hardware settings. Please make sure your

adapter is set to match the driver settings. At any

time you can click on the "Resources" tag to change

settings.

# If the Adapter is running properly, cancel out to the

control panel group.

# If problems persist please contact D-Link Technical

support for help.

Reported Bugs (B=Bug/S=Solution)

B: DE-600 will not see network.

S: Many new laptops cannot use the DE-600 due to thier

advanced Parallel port designs. The most stable

method will be using ODI irregardless if you are using

netware. It may be necessary to purchase an upgrade to

a DE-620 which has a full driver set and works with most

current laptop.

B: DE-650C or DE-650 T is recogized but the driver will

not work.

S: The drivers built into '95 are for the DE-650CT. We are

working on drivers which should be ready in mid-OCT'95.

The NDIS 2.0 drivers cannot be used since the PCMCIA

support loads after load the real mode network drivers.

B: Some DE-650CT will not work with the built in drivers.

S: Due to engineering changes it is best to use the drivers

from DLKWIN95.EXE for the 650CT.

Appendix for Support Access

TechFax +44 (0)181-235 5511

FTP: ftp.dlink.com (192.152.81.48) 7days/24hrs

WWW: www.dlink.com (192.152.81.48) 7days/24hrs