DIAGS IBM PC Diagnostic Error Codes

Code  Description	  (Diagnostic Error Codes)

1x   undetermined problem errors

2x   power supply errors

61   battery error (PS/2)

62   configuration changed but no adaptors changed or CRC error

63   memory size during POST does not match ~CMOS~ RAM

65   card Id's in CMOS do not match system

1xx   System Board Errors

101   system board failed

102   BIOS ROM checksum error (PC, XT), Timer (AT)

103   BASIC ROM checksum error (PC, XT), Timer interrupt (AT)

104   interrupt controller (PC, XT), Protected mode (AT)

105   timer (PC,XT), Last ~8042~ command not accepted (AT)

106   converting logic test failure

107   adapter card or math coprocessor (NMI)

108   timer bus test

109   DMA test error

110   system board memory error (PS/2)

111   adapter memory

112   system unit adapter failure

114   system unit and adapter card unit failure (PS/1)

121   unexpected hardware interrupt

131   cassette wrap test failed

151   real-time clock, CMOS RAM or battery

152   real-time clock

161   system options error, battery failure

162   CMOS RAM configuration error

163   CMOS time and date not set

164   system memory configuration is incorrect

165   adapter added/removed

199   user indicated configuration not correct

2xx   RAM Errors

201   memory test error

202   memory address error (Line error	0 - 15)

203   memory address error (Line error 16 - 23)

216   motherboard memory

3xx   Keyboard Errors

301   keyboard did not respond to software reset or a stuck

(number preceding 301 is scan code for stuck key)

302   user indicated keyboard error or AT system unit is locked

303   keyboard or system board error

304   keyboard or system board error; CMOS does not match system

341   replace keyboard

342   replace keyboard interface cable

343   replace enhancement card or cable

365   keyboard (replace keyboard)

366   interface cable (replace cable)

367   enhancement card or cable (replace)

4xx   Monochrome Monitor Errors

401   monochrome memory test, horizontal sync frequency

test, or video test failed

408   user indicated display attributes failure

416   user indicated character set failure

424   user indicated 80x25 mode failure

432   parallel port test failed (monochrome adapter)

5xx   Color Monitor Errors

501   CGA memory test failed, horizontal sync frequency test, or

video test failed

508   user indicated display attribute failure

516   user indicated character set failure

524   user indicated 80x25 mode failure

532   user indicated 40x25 mode failure

540   user indicated 320x200 graphics mode failure

548   user indicated 640x200 graphics mode failure

556   light pen test

564   user indicated screen paging test

6xx   Diskette Drive/Adapter Errors

601   diskette power on diagnostics test failed

602   diskette test failed (boot record not valid)

603   diskette size error

606   diskette verify function failed

607   write protected diskette

608   bad command diskette status returned

610   diskette initialization failed

611   time-out - diskette status returned

612   bad NEC ~FDC~ - diskette status returned

613   bad DMA - diskette status returned

614   DMA boundary error

621   bad seek - diskette status returned

622   bad CRC - diskette status returned

623   record not found - diskette status returned

624   bad address mark - diskette status returned

625   bad NEC seek - diskette status returned

626   diskette data compare error

627   diskette change line error

628   diskette removed

7xx   8087 Math Coprocessor

701   coprocessor test failure

9xx   Parallel Printer Adapter Errors

901   parallel printer adapter test failed

10xx  Alternate Parallel Printer Adapter

1001  alternate printer adapter test failure

11xx  Asynchronous Communications Adapter Errors

1101  asynchronous communications adapter test failed

1110  modem status register not clear

1111  ring-indicate

1112  trailing edge ring-indicate

1113  receive and delta receive line signal detect

1114  receive line signal detect

1115  delta receive line signal detect

1116  line control register: all bits cannot be set

1117  line control register: all bits cannot be reset

1118  transmit holding and/or shift register stuck on

1119  data ready stuck on

1120  interrupt enable register: all bits cannot be set

1121  interrupt enable register: all bits cannot be reset

1122  interrupt pending stuck on

1123  interrupt ID register stuck on

1124  modem control register: all bits cannot be set

1125  modem control register: all bits cannot be reset

1126  modem status register: all bits cannot be set

1127  modem status register: all bits cannot be reset

1128  interrupt ID

1129  cannot force overrun error

1130  no modem status interrupt

1131  invalid interrupt pending

1132  no data ready

1133  no data available interrupt

1134  no transmit holding interrupt

1135  no interrupts

1136  no received line status interrupt

1137  no receive data available

1138  transmit holding register not empty

1139  no modem status interrupt

1140  transmit holding register not empty

1141  no interrupts

1142  no IRQ4 interrupt

1143  no IRQ3 interrupt

1144  no data transferred

1145  maximum baud rate

1146  minimum baud rate

1148  time-out error

1149  invalid data returned

1150  modem status register error

1151  no DSR and delta DSR

1152  no DSR

1153  no delta DSR

1154  modem status register not clear

1155  no CTS and delta CTS

1156  no CTS

1157  no delta CTS

12xx  Alternate Asynchronous Communications Adapter

1200-1257 same as 1100-1157

13xx  Game Control Adapter Errors

1301  game control adapter test failed

1302  joystick test failed

1380  audio card and joystick (PS/1)

14xx  Printer Errors

1401  printer test failed

1404  matrix printer failed

15xx  SDLC Communications Adapter Errors

1501  adapter test failure

1510  8255 port b failure

1511  8255 port a failure

1512  8255 port c failure

1513  8253 timer 1 did not reach terminal count

1514  8253 timer 1 stuck on

1515  8253 timer 0 did not reach terminal count

1516  8253 timer 0 stuck on

1517  8253 timer 2 did not reach terminal count

1518  8253 timer 2 stuck on

1519  8273 port b error

1520  8273 port a error

1521  8273 command/read time-out

1522  interrupt level 4 failure

1523  ring Indicate stuck on

1524  receive clock stuck on

1525  transmit clock stuck on

1526  test indicate stuck on

1527  ring indicate not on

1528  receive clock not on

1529  transmit clock not on

1530  test indicate not on

1531  DSR not on

1532  CD not on

1533  CTS not on

1534  DSR stuck on

1535  CD stuck on

1536  CTS stuck on

1537  level 3 interrupt failure

1538  receive interrupt results error

1539  wrap data compare error

1540  DMA channel 1 error

1541  DMA channel 1 error

1542  error in 8273 error checking or status reporting

1547  stray interrupt level 4

1548  stray interrupt level 3

1549  interrupt presentation sequence time-out

16xx  Display Emulation Errors (327x, 5520, 525x)

17xx  Fixed Disk Errors

1701  post error

1702  adapter error

1703  drive error (seek)

1704  adapter or drive error

1705  no record found

1706  write fault error

1707  track 0 error

1708  head select error

1709  defective error check

1710  read buffer overrun

1711  bad address mark

1712  undetermined error

1713  data compare error

1714  drive not ready

1780  disk 0 failure

1781  disk 1 failure

1782  disk controller failure

1790  fixed disk 0 error

1791  fixed disk 1 error

18xx  I/O Expansion Unit Errors

1801  I/O expansion unit POST error

1810  enable/disable failure

1811  extender card wrap test failed (disabled)

1812  high order address lines failure (disabled)

1813  wait state failure (disabled)

1814  enable/disable could not be set on

1815  wait state failure (enabled)

1816  extender card wrap test failed (enabled)

1817  high order address lines failure (enabled)

1818  disable not functioning

1819  wait request switch not set correctly

1820  receiver card wrap test failure

1821  receiver high order address lines failure

19xx  3270 PC Attachment Card Errors

20xx  Bisync Communications Adapter Errors

2010  8255 port a failure

2011  8255 port b failure

2012  8255 port c failure

2013  8253 timer 1 did not reach terminal count

2014  8253 timer 1 stuck on

2016  8253 timer 2 didn't reach terminal count or timer 2 stuck on

2017  8251 DSR failed to come on

2018  8251 CTS not sensed

2019  8251 DSR stuck on

2020  8251 CTS stuck on

2021  8251 hardware reset failed

2022  8251 software reset failed

2023  8251 software error reset failed

2024  8251 transmit ready did not come on

2025  8251 receive ready did not come on

2026  8251 could not force "overrun" error status

2027  interrupt failure, no timer interrupt

2028  transmit interrupt, card or planar failure

2029  transmit interrupt, card failure

2030  receive interrupt, card or planar failure

2031  receive interrupt, card failure

2033  ring indicate stuck on

2034  receive clock stuck on

2035  transmit clock stuck on

2036  test indicate stuck on

2037  ring indicate stuck on

2038  receive clock not on

2039  transmit clock not on

2040  test indicate not on

2041  DSR not on

2042  CD not on

2043  CTS not on

2044  DSR stuck on

2045  CD stuck on

2046  CTS stuck on

2047  unexpected transmit interrupt

2048  unexpected receive interrupt

2049  transmit data did not equal receive data

2050  8251 detected overrun error

2051  lost DSR during data wrap

2052  receive time-out during data wrap

21xx  Alternate Bisync Communications Adapter Errors

2100-2152 same as 2000-2052

22xx  Cluster Adapter Errors

24xx  EGA Errors

2401  adapter test failure

2408  user-indicated display attributes

2416  user-indicated character set

2424  user-indicated 80x25 mode

2432  user-indicated 40x25 mode

2440  user-indicated 320x200 graphics mode

2448  user-indicated 640x200 graphics mode

2456  light pen test

2464  user-indicated screen paging test

2501  EGA Display Error

28xx  3278/79 Emulator Adapter (PC, XT)

29xx  Color Matrix Printer Errors

30xx  Local Area Network Adapter

31xx  Alternate Local Area Network Adapter

33xx  Compact Printer Errors

36xx  IEEE 488 Adapter

37xx  Reserved for Future Use

38xx  Data Acquisition Adapter

39xx  PGA Display and/or Adapter

48xx  Internal Modem

49xx  Alternate Internal Modem

71xx  Voice Communications Adapter

73xx  3.5 Inch Diskette Drive

7301  diskette drive/adapter test failure

7306  diskette change line error

7307  write-protected diskette

7308  bad command

7310  track zero error

7311  timeout

7312  bad NEC7313 bad DMA

7314  DMA boundary error

7315  bad index

7316  speed error

7321  bad seek

7322  bad CRC

7323  record not found

7324  bad address mark

7325  bad NEC seek

86xx  PS/1 Mouse Error

- All error codes for the diagnostic and advanced

diagnostic packages for the PC,  XT and  AT are

represented with the device number followed  by

two digits other than 00.

The device number plus 00 represents successful

completion of the test.