| S3 Speed Up  Version 3.02

|

| Increases the transfer rate of S3 graphics cards.

|

| Copyright (c) 1994,96 Dietmar Meschede.  All rights reserved.

S3 Speed Up is a TSR program, which speeds up VGA mode 13h (see below!)

and most VESA modes of S3 graphics cards (S3 928 or newer).

S3SPDUP /? shows a help screen.

S3 Speed Up needs a S3 VESA graphics card (S3 928 or newer) to run.

The program doesn't check, if a S3 graphics card is present!

S3 Speed Up was designed to work with S3 928 chipsets.

It should work with newer S3 chipsets (Trio32, Trio64, Trio64V+,

Vision 864/964, Vision 868/968, Virge, Virge/VX, ...), too.

The speed up of VGA mode 13h (320x200x256) is not 'Mode X' compatible.

This means that some programs (for example DOOM) will not work when

the VGA mode speed up is enabled.

Programs using VESA VBE/Core 2.0 linear frame buffer modes (for example

QUAKE) can not profit from S3 Speed Up.  It is unnecessary to install

S3 Speed Up, if S3 VBE/Core 2.0 version 3.10 or above is installed.

How does S3 Speed Up work?  It activates 'Enhanced Mode Mappings' and

enables 'Linear Addressing' in VGA mode 13h.  And it activates 'Linear

Addressing' in most VESA modes (not in text and 4 bits/pixel modes).

This package consists of the following 5 files:

FILE_ID.DIZ   Short description.

S3SPDUP.DOC   This file.

S3SPDUP.DOK   German description.

S3SPDUP.EXE   S3 Speed Up.

RESULTS.TXT   Contains test results demonstrating the effect of S3 Speed Up.

The program and the documentation in this package are copyrighted

by Dietmar Meschede.  All rights reserved.

This package is Freeware.  You may use, copy and distribute this

package, under the following conditions:

- Don't modify the files in this package in any way.

- Only distribute the original package, with all files included.

YOU ARE USING THESE PROGRAMS AT YOUR OWN RISK!  THE AUTHOR

(DIETMAR MESCHEDE) IS NOT LIABLE FOR ANY DAMAGE CAUSED BY

THE USE OF THESE PROGRAMS.  IF YOU ARE NOT SURE ABOUT THIS,

OR IF YOU DON'T ACCEPT THIS, THEN DO NOT USE THESE PROGRAMS.

Dietmar Meschede

F�rstenberger Stra�e 50

D-49479 Ibbenb�ren

EMail:         mesched@math.uni-muenster.de

FTP Directory: ftp://ftp.uni-muenster.de/pub/msdos/mesched

WWW Homepage:  http://wwwmath.uni-muenster.de/~mesched