The 6845 cathode ray tube controller is	used on	the PC for both

monochrome and color video systems.  The PS/2 VGA system provides some

emulation for both of these controllers	at the hardware	level.

The 6845 controller uses four I/O ports	3B4H, 3B5H, 3B8H, and 3BAH on

the monochrome card (substitute	'D' for	'B' for	color card).

Port 3B4H is the 6845 index register, to which a control register value

of 0 to	17 is sent before a read/write to the data register 3B5H.

The 6845 has 18	control	registers, 0-17.  The first ten	fix the

horizontal and vertical	display	parameters.  Incorrect settings	of

registers 1-9 can damage a monitor.  Registers 10 and 11 set the shape

of the cursor; 14 and 15 control its location.	Registers 12 and 13 can

handle scrolling. Numbers 16 and 17 report light pen position.

Registers 12-15	are read/write.	 Registers 16-17 are read only.	All

other registers	are write-only.

6845 Internal Registers:	VGA Emulation and Extension:

R0:  Horizontal	total characters	(Total characters less 5)

R1:  Horizontal	displayed characters	(Display char./line -1)

R2:  Start Horizontal blanking

R3:  End Horizontal blanking

R4:  Vertical total lines		Start Hor. Retrace Pulse

R5:  Vertical total adjust raster	End Hor. Retrace

R6:  Vertical display line		Vert. Total -2 (low 8 bits)

R7:  Vertical sync position line	Overflow  (see below)

R8:  Interlace:				Preset Row Scan	(see below)

00		  10=non-interlace

01=duplicate 11=different

R9:  Maximum raster address		Max. Scan Line (see below)

R10: Cursor Start raster

R11: Cursor End	raster

R12: Start address high

R13: Start address low

R14: Cursor high

R15: Cursor low

R16: Light pen high			Vertical Retrace Start

R17: Light pen low			Vertical Retrace End (see below)

R18:					Vertical Display Enable	End

R19:					Underline Location

R20:					Start Vertical Blank

R21:					End Vertical Blank

R22:					CRTC Mode Control

R23:					Line Compare

The VGA	emulation of the 6845 allows all registers to be read/write.

Port 3B8H is a CRT control port:

Bit	 Function	      PS/2 Emulation: None

7-6	 Reserved

5	 Blink enable

4	 Reserved

3	 Video enable

2-1	 Reserved

0	 High resolution mode

Port 3BAH is a CRT read/only status port and a write/only feature

control	port on	the PS/2.  As a	read/only port:

Bit	 Function	     PS/2 Extension: Input Status Register 1

7-6	 Reserved	     Reserved

5	 Reserved	     Attribute controller diagnostic 0

4	 Reserved	     Attribute controller diagnostic 1

3	 Video dots	     Vertical retrace

2-1	 Reserved	     Reserved

0	 Horizontal sync     Display enable (1=hor. or vert. retrace)

As a write/only	port on	the PS/2, the Write Feature Control Register,

all bits are reserved (bit 3 must be 0).

On the Hercules	Graphics Controller, bit 7 may be used to distinguish

a Hercules card	from an	IMB Monochrome or Color	Adapter.  On the

Hercules card, bit 7 goes 0 on vertical	retrace	(50 Hz).  On the

IBM card, bit 7	does not change.  The Hercules and the Hercules	Plus

can be distinguished with bits 4 and 5 (1 and 0	for Plus).

____________________________________________________________________________

----------------------------------------------------------------------------

Ports: 3C0H-3CFH: VGA Support

----------------------------------------------------------------------------

In addition to emulation for the 6845 status, index, and control ports,

the VGA	system on the PS/2 uses	the ports 3C0H-3CFH for	additional video

status information and control.

Input Status Register 1:       3BAH or 3DAH: (R)

Bit	 Function

7-6	 Reserved = 0

5-4	 Diagnostic 0,1	 Selectively connected to two of eight

3	 Vertical Retrace

2-1	 Reserved = 0

0	 Display Enable

Attribute Registers:	       3C0H-3C1H:

0

Attribute Controller Registers:

Bit	 Function

7-6	 Reserved = 0

5	 Palette Address Source

(Set 0 when loading color palette registers)

4-0	 Attribute Address

Each attribute data register is	written	at 3C0H	and read from 3C1H.  To

initialize the address flip-flop, issue	IOR to 3BAH or 3DAH.  Then load

the attribute controller register. This	toggles	the flip-flop for a OUT

to the indexed data register. The flip-flop is not toggled by a	read

from 3C1H.

Palette	Registers: Index 00 to 0FH:

Bit	 Function

7-6	 Reserved = 0

5-0	 P5-P0	Used to	map color input	to display color

Attribute Mode Control Register: Index 10H:

Bit	 Function

7	 P5, P4	Select 1=source	from bits 1,0 of Color Select Register

6	 PEL Width - 1 for 256-color mode

5	 PEL Panning Compatibility

4	 Reserved = 0

3	 Select	Background Intensity

2	 Enable	Line Graphics Character	Code (0=ninth dot same as backgnd)

1	 Mono Emulation

0	 Graphics/Alphanumeric Mode (1=graphics)

Overscan Color Register: Index 11H:

Bit	 Function

7-0	 P7-P0 Border color

Color Plane Enable Register: Index 12H:

Bit	 Function

7-6	 Reserved = 0

5-4	 Video Status MUX - Selects 2 of 8 color outputs for status port

3-0	 Enable	Color Plane

Horizontal PEL Panning Register: Index 13H:

Bit	 Function

7-4	 Reserved = 0

3-0	 Horizontal PEL	Panning	(number	of pixels to pan)

Color Select Register: Index 14H:

Bit	 Function

7-4	 Reserved = 0

3-2	 S_color76 - two high-order bits of 8 bit color	value

2-0	 S_color54 - replaces P5 and P4	in Attrib.Palette Reg.

Read Input Status Register 0:  3C2H: (R)

Bit	 Function

7	 CRT interrupt 1 = vertical retrace interrupt pending

6-5	 Reserved

4	 Switch	Sense Bit: Lets	POST determine monochrome or color

3-0	 Reserved

Write Misc. Output Register:   3C2H: (W)

Read  Misc. Output Register:   3CCH: (R)

Bit	 Function

7	 Vert. sync polarity 0 = positive retrace

6	 Hor. sync polarity  0 = positive retrace

bits	7,6= 1 0 for 400 lines

0 1 for 350 lines

1 1 for 480 lines

5	 Page bit for odd/even (dianostic use) 1 = high	64K page

4	 Reserved = 0

3-2	 Clock select

0 0 =	25.175 MHz for 640 hor.	pixels

0 1 =	28.322 MHz for 720 hor.	pixels

1 0 =	external clock at aux. video input (14.3-28.4 MHz)

1 1 =	reserved

1	 Enable	RAM 0 =	disable	video RAM address decode from CPU

0	 I/O address select - CRTC I/O 0 = 3BxH, 1 = 3DxH

Video Subsystem	Enable:	       3C3H:

Bit	 Feature

7-1	 Reserved

0	 Video subsystem enable: 1 = video I/O and memory address

decoding is enabled.

This register is not affected by the VGA sleep bit (102H bit 0).

Sequencer Registers:	       3C4H-3C5H:

Sequencer Address Register:    3C4H:

This register is loaded	with a index to	the following Sequence

Data registers:

Sequence Data Registers:       3C5H:

Reset Register (R/W) (Index 0):

Bit	 Function

7-2	 Reserved

1	 Synchronous reset 0 = synchr. clear and halt (before Clocking

Mode register bit	0 or Misc. Output Register bit 2)

0	 Asynchronous reset 0 =	asynchr. clear and halt

Clocking Mode Register (R/W) (Index 1):

Bit	 Function

7-6	 Reserved = 0

5	 Screen	off  1 = screen	off (use for rapid full-screen update by

giving CPU maximum memory bandwidth)

4	 Shift 4  0 = video serializers	are loaded every char. clock,

1 = video serializers	loade every fourth clock (use

with 32 bit fetches/cycle)

3	 Dot clock 0 = select normal dot clock,	1 = master clock/2

(clock/2	used for 320 and 360 hor. pixel	modes)

2	 Shift load: if	0 and if bit 4=0, video	serializers reloaded

every char. clock, when 1, every	other char. clock

(use with 16 byt	fetches/cycle)

1	 Reserved = 0

0	 8/9 dot clocks	0 = char. clocks 9 dots	wide.

Map Mask Register (R/W)	(Index 2):

Bit	 Function

7-4	 Reserved = 0

3	 Map 3 enable 1	= CPU can write	to map 3

2	 Map 2 enable

1	 Map 1 enable

0	 Map 0 enable

If this	register is set	to 0FH,	the system microprocessor can perform

32 bit wide write in only one memory cycle.

Character Map Select Register (R/W) (Index 3):

Bit	 Function

7-6	 Reserved = 0

5	 Character Map select high bit A

4	 Character Map select high bit B

3-2	 Character Map select A

1-0	 Character Map select B

In alphanumeric	modes, bit 3 of	the attribute byte normally is used

to control foreground intensity.  This bit may be redefined, however,

to switch between character sets.  For this feature to be enabled,

the following must be true:

Memory Mode register bit 1 = 1

Character Map	Select A is not	the same as Character Map Select B

If either is not true, the first 16K of	Map 2 is used.

For selection A:

Bit  5	 3   2	 Map  Table Location

0	 0   0	  0   1st 8K of	Map 2

0	 0   1	  1   3rd 8K of	Map 2

0	 1   0	  2   5th 8K of	Map 2

0	 1   1	  3   7th 8K of	Map 2

1	 0   0	  4   2nd 8K of	Map 2

1	 0   1	  5   4th 8K of	Map 2

1	 1   0	  6   6th 8K of	Map 2

1	 1   1	  7   8th 8K of	Map 2

Similarly for selection	B using	bits 4,	1, and 0.

Memory Mode Register (R/W) (Index 4)

Bit	 Function

7-4	 Reserved = 0

3	 Chain 4   0 = enable CPU to access data at addresses within

bit map using Map Mask register.

1 = enable CPU to access data at addresses according

to two low order	bits of	address	A1, A0:

00=map 0, 01=map 1, 10=map 2, 11=map 3.

2	 Odd/even  0 = use maps	0,2 or 1,3 according to	parity of address.

1 = access data sequentially	using Map Mask register

1	 Extended memory 1 = greater than 64K video memory present

0	 Reserved = 0

Digital	to Analog Converter Registers: 3C6H-3C9H:

3C6H R/W:   Pixel Mask (color	look-up	table destroyed	on write)

3C7H Read:  DAC State	Register

3C7H Write: Pixel Address

3C8H R/W:   Pixel Address

Read Feature Control Register: 3CAH: All bits reserved.

Miscellaneous Output Register: 3CCH (R)	 See port 3C2h.

Graphics Registers:	       3CEH-3CFH:

Graphics Controller Registers: 3CEH:

This read/write	register is loaded with	the index to the graphic registers

described below:

Graphics Registers (R/W)       3CFH:

Set/Reset Register (R/W) (Index	0):

Bit	 Function

7-4	 Reserved = 0

3	 Set/Reset Map 3

2	 Set/Reset Map 2

1	 Set/Reset Map 1

0	 Set/Reset Map 0

Enable Set/Reset Register (R/W)	(Index 1):

Bit	 Function

7-4	 Reserved = 0

3	 Enable	Set/Reset Map 3

2	 Enable	Set/Reset Map 2

1	 Enable	Set/Reset Map 1

0	 Enable	Set/Reset Map 0

Color Compare Register (R/W) (Index 2):

Bit	 Function

7-4	 Reserved = 0

3	 Color Compare Map 3

2	 Color Compare Map 2

1	 Color Compare Map 1

0	 Color Compare Map 0

Data Rotate Register (R/W) (Index 3):

Bit	 Function

7-5	 Reserved = 0

4-3	 Function Select  00 Data unmodified, 01 ANDed,	10 ORed, 11 XORed

2-0	 Rotate	Count for right-rotate (write mode 0)

ReadMap	Select Register	(R/W) (Index 4):

Bit	 Function

7-2	 Reserved = 0

1-0	 Map Select for	read

Graphics Mode Register (R/W) (Index 5):

Bit	 Function

7	 Reserved = 0

6	 256 color mode:

0=allow bit 5	to control loading of Shift registers

5	 Shift Register	Mode:

1=format serial data with even-numbered bits for even	maps

odd-numbered bits for odd maps

4	 Odd/Even:  1=odd/even addressing mode

3	 Read Type: 0=reads from memory	map selected by	Read Map Select	Reg.

2	 Reserved

1-0	 Write Mode for	memory map:

00=data rotated unless Set/Reset is enabled

01=from contents of system CPU latches

10=map n (0-3) filled	with 8 bits of data bit	n

11=from 8 bits in Set/Reset register for that	map

Miscellaneous Register (R/W) (Index 6):

Bit	 Function

7-4	 Reserved = 0

3-2	 Memory	Map: 00=A0000 for 128K bytes

01=A0000 for  64K bytes

10=B0000 for  32K bytes

11=B8000 for  32K bytes

1	 Odd/Even: 1=use odd/even maps for odd even addresses

0	 Graphics Mode:	1=graphics mode, 0=alphanumeric	mode

Color Don't Care Register (R/W) (Index 7):

Bit	  Function

7-4	 Reserved = 0

3	 Map 3 - Don't Care (0=Don't participate in color compare cycle)

2	 Map 2 - Don't Care

1	 Map 1 - Don't Care

0	 Map 0 - Don't Care

Bit Mask Register (R/W)	(Index 8):

Bit	 Function

7-0	 Mask: 0=bit n in each map to be immune	to change (modes 0 and 2)