Setting Up The PPPoE Dialer Under Mac OS X

Please go through the following steps on how to set up the PPPoE Dialer under Mac OS X: -

- Power on your Macintosh, before this, please make sure that all the physical connection between your modem and your Macintosh have been established.

- Click the APPLE icon on the most left-hand corner of the desktop.

- Scroll down to locate the SYSTEM PREFERENCES option.

- Click on the NETWORK icon on the first row or under the INTERNET & NETWORK row.

- Select ‘Automatic’ for the LOCATION option.

- Select ‘Built-In Ethernet’ for the SHOW option. (You will noticed that the tabs below will be changed accordingly)

- Key-in the following configurations under the TCP/IP tab: -

  - Select ‘Manually’ for CONFIGURE option.

  - Configure ’192.168.1.2’ for IP ADDRESS field.

  - Configure ‘255.255.255.0’ for SUBNET MASK field.

  - Configure ’192.168.1.1’ for ROUTER field.

  - Configure ‘165.21.83.88’ and ‘165.21.100.88’ for Domain Name Servers field.

  - Click the ‘Apply Now’ button to save the changes.

- Go to PPPoE tab, tick the ‘Connect using PPPoE’ option, and key-in the following entries under PPPoE tab: -

  - Configure ‘SingNet BroadBand’ or ‘Pacific Internet’ for SERVICE PROVIDER field if you are a Singnet subscriber or PacNet subscriber respectively.

  - Configure ‘SingNet BroadBand’ or ‘Pacific Internet’ for PPPoE SERVICE NAME field if you are a Singnet subscriber or PacNet subscriber respectively.

  - Configure ‘username@singnet’ or ‘username@pacific.net.sg’ for ACCOUNT NAME field if you are a Singnet subscriber or PacNet subscriber respectively.

  - Configure your password for PASSWORD field.

  - Click the ‘PPPoE Options’ button, uncheck the ‘Disconnect if idle for 15 minutes’ option, and then click ‘OK’ button to continue.

  - Then tick the ‘Show PPPoE status in menu bar’ option.

  - Finally, click the ‘Apply Now’ button to save the changes.

- Then, click the ‘Close’ icon to close the SYSTEM PREFERENCES window.

- Double-click the ‘Untitled’ icon on the desktop, and then double-click again the ‘Applications’ icon.

- Under the APPLICATIONS  window, double-click the ‘Internet Connect’ icon, and then the INTERNAL MODEM window will pop-up.

- Select the ‘Built-In Ethernet’ for CONFIGURATION option. (You will noticed that the INTERNAL MODEM window will be changed to BUILT-IN ETHERNET window)

- Finally, click the ‘Connect’ button to connect to your respective ISP server.