November 1992

3Com 3+ Share Driver Installation

for

D-Link Ethernet Pocket LAN Adapter

==================================

The D-Link 3COM 3+ Share Driver Program lets you run 3Com 3+ Share

on your PC with the D-Link Ethernet Pocket LAN Adapter.

The following files will be used in this installation:

Files            Contents

-----------      -------------------------

DE600PD.SYS      DE-600 Driver Program

ETH.SYS          3Com 3+ Share Driver

1. Follow the installation procedure for 3Com EtherLink board.

2. Copy the program ETH.SYS from the Installation Diskette for

the D-Link Ethernet Pocket LAN Adapter to the boot disk/diskette.

This program will replace the program of the same file name

(ETH.SYS) when you perform the installation procedure in step 1

above.

3. In the CONFIG.SYS file on the boot disk/diskette, make sure

you have the following two statements.  The order of these two

statement must be as shown below.

DEVICE = DE600PD.SYS <packet_int_no>

DEVICE = ETH.SYS

You may now boot and start your 3COM workstation.

Notes:

<packet_int_no> is the software interrupt vector number used by the

packet driver program, which can be from 96 to 128.