HARDWARE CONFLICT TESTS - JUMBO & TRAKKER

rev 9/93

If you have tried a clean boot and are still having hardware

conflicts, try the following tests:

1.   Use the turbo button, or look up the keyboard turbo function

for your computer in the manual to take your system out of

turbo mode.  If this resolves the problem, then your system

bus is probably running too fast for your computer to do dual

DMA (direct memory access) during tape backup operations, or

the bus speed is just too fast for some of the cards in your

system.  Slowing bus speed by using the turbo switch will also

slow down your processor speed.  If your tape operation

operates properly with the system slowed down, this indicates

that you are a good candidate for fixing the problem by

resetting the bus to a slower speed in the CMOS setups.  (You

can access your CMOS setups on system start up or some

companies provide setup diskettes with their computers.)

2.   Try Non-Concurrent Disk/Tape Operation.  Non-concurrent

operation means that instead of trying to do a difficult dual

DMA operation, the Colorado software will use a much easier

single DMA transfer method.  Difficulty with dual DMA

operations can be related to either the system board

(Motherboard) of the floppy controller card.  Replacing the

floppy controller, or using a dedicated Tape Controller Card

(Colorado Memory Systems model FC-10) are alternatives if the

current floppy controller is causing the problem.

3.   Certain 486 computers have processor caches that can interfere

with tape performance.  Software versions starting with 3.XX

will allow you to disable the processor cache on 486 systems

while the tape software is running.  To disable caching on 486

systems, use the following command line operation while you

are in the tape sub directory (i.e. at the C:\TAPE prompt).

TAPE SETUPS /Y/W