Premiere/PCI II Platforms

Phase II BIOS User’s Guide

Version 2.0

August, 1994

Table of Contents

Setup Overview

Introduction

Command Screen Navigation

Main Menu Navigation

Menus

Main Menu

System Date

System Time

Floppy A:

Floppy B:

Hard Disk C:

Hard Disk D: ....

Disk drive assignments

Hard Disk Sub-Menu

Hard Disk Type

Maximum Capacity

Initialization Timeout

IDE Translation Mode

Multiple Sector Setting

Fast Programmed I/O Modes

Main Menu Continued

Language

Boot Options

Boot Options Sub-Menu

Boot Sequence

System Cache

Boot Speed

Num Lock

Setup Prompt

Hard Disk Pre-Delay

Typematic Rate Programming

Typematic Rate Delay

Typematic Rate

Main Menu Continued

Video Mode

Mouse

Base Memory

Extended Memory

Advanced Menu

Processor Type

Processor Speed

Cache Size

Peripheral Configuration

Advanced Chipset Configuration

Power Management Configuration

Plug and Play Configuration

Peripheral Configuration Sub-Menu

Configuration Mode

PCI IDE Interface

Standard ISA IDE Interface

Floppy Interface

Serial Port 1 Address

Serial Port 2 Address

Parallel Port Address

Parallel Port Mode

Serial Port 1 IRQ

Serial Port 2 IRQ

Parallel Port IRQ

Advanced Chipset Configuration Sub-Menu

Base Memory Size

ISA Bus Speed

Byte Merging

PCI IDE Prefetch Buffers

ISA LFB Size

ISA LFB Base Address

Video Palette Snoop

Latency Timer (PCI Clocks)

Power Management Configuration Sub-Menu

Advanced Power Management

IDE Drive Power Down

VESA Video Power Down

Inactivity Timer (Minutes)

Hot Key

Plug and Play Configuration Sub-Menu

Configuration Mode

ISA Shared Memory Size

ISA Shared Memory Base Address

Security Menu

User Password is

Administrative Password is

Set User Password

Set Administrative Password

Unattended Start

Security Hot Key (CTRL-ALT-)

Exit Menu

Exit Saving Changes

Exit Discarding Changes

Load Setup Defaults

Discard Changes

# Setup Overview

## Introduction

This User’s Guide documents the “Phase II” BIOS Setup Utility.  It explains the overall organization of the utility, how to navigate through the utility, and individual fields and options.

The “Phase II” BIOS Setup Utility is divided into 4 major menus:

- Main Menu

- Advanced Menu

- Security Menu

- Exit Menu

Each of the major menus occupy the left and center sections of the computer screen.  Selecting certain fields within the major menus will drop the user into sub-menus.  Displayed on the right side of the screen  is a Command Screen which describes the commands used to interact with the Setup Utility.  The following section documents these commands.  The subsequent four sections describe each of the main menus.  Sub-menus are documented within the main menus from which they are invoked   In each section the field which is being displayed is bold type.  If there are user selectable options for that field they are displayed in italic  type.  Explanations and comments about the fields and options are contained within boxes.

Explanations and comments here

## Command Screen Navigation

The right side of the Setup screen contains a list of  commands which are available to the user as they navigate through and interact with the Setup Utility.  These commands are displayed at all times, for each menu and sub-menu.

#### F1	Help

Help is provided for a subset of the fields.  It can not be invoked when parameter options are being displayed.

#### ESC	Back

The ESC key provides a mechanism for backing out of any field.  When the ESC key is entered from the top level of any field, the field options cease to be displayed.  When the ESC key is entered from the top level of any sub-menu, the parent menu is re-entered.  When the ESC key is entered from the top level of any major menu the following screen is displayed:

Exit Discarding Changes

Press Enter to Continue

Press ESC to Abort

At this point pressing the Enter key will cause the user to leave the Setup Utility without saving any changes they might have made.  Pressing the ESC key will  bring them back to the top level of the current menu.

#### Enter	Select

The Enter key is used to activate sub-menus, change parameters and select parameter options.

#### Previous Item

The up arrow is used to move to the menu item above the current item.  It does not wrap around.

#### ¯	Next Item

The down arrow is used to move to the menu item below the current item.  It does not wrap around.

#### «	Select Menu

The left and right arrow keys are used to move between the 4 major menus. They do not wrap around.

#### F5	Setup Defaults

The Setup Defaults key is used to set all Setup Parameters to their default values.  When this key is pressed the following message appears:

Load Setup Defaults?

Press Enter to Continue

Press ESC to Abort

If the Enter key is selected then all Setup Parameters will be set to their default values.  If the ESC key is selected, then the user is returned to where they were before F5 was pressed without effecting any change.

#### F6	Previous Values

The Previous Values key is used to set all Setup Parameters to their previous values.  When this key is pressed the following message appears:

Discard Changes?

Press Enter to Continue

Press ESC to Abort

If the Enter key is selected then all Setup Parameters will be return to the values selected when Setup was entered.  If the ESC key is selected, then the user is returned to where they were before F6 was pressed without effecting any change.

#### F10	Save & Exit

The Save & Exit key is used to exit the Setup Utility saving the current state of all parameters   When this key is pressed the following message appears:

Exit Saving Changes?

Press Enter to Continue

Press ESC to Abort

If the Enter key is selected then the Setup Utility will be exited with all parameters set to their current values.  If the ESC key is selected, then the user is returned to where they were before F10 was pressed without effecting any change.

## Main Menu Navigation

Each menu  contains a number of parameters.  Most parameters can be changed by the user, although some are informative only.  Parameters which can be changed by the user are displayed in black, informative only parameters are displayed in blue.  Depending on the security option chosen and in effect (via password), a parameter can be either changeable or non-changeable.  A parameter which is non changeable due to insufficient security privilege or other reasons will be grayed out.  Parameters which have no effect based on the value other parameters are not displayed.

# Menus

## Main Menu

#### System Date

Month-Day-Year

The System Date field specifies the current date .

#### System Time

Hour:Minute:Second

The System Time  field specifies the current time.

#### Floppy A:

#### Floppy B:

Disabled

360 Kb, 5.25 “

1.2 Mb, 5.25”

720Kb, 3.5"

1.44 Mb, 3.5”

2.88 Mb, 3.5"

The Floppy A: Type and Floppy B: Type fields specify the physical size and maximum supported media size of the floppy drives attached to the floppy interface.

#### Hard Disk C:

#### Hard Disk D: ....

Identification string returned from drive

Not Installed

The Hard Disk C: through Hard Disk F: fields report the presence of, and identification string reported by, up to 4 physical IDE drives (two each of PCI and ISA.  When the cursor is placed on one of these fields and it is selected by hitting the Enter Key, the Hard Disk Sub-Menu appears (see below) allowing parameters which affect the drives to be set.

#### Disk drive assignments

Drive letters will be assigned to physical drives as follows:

| PCI IDE Interface Only |  |
| --- | --- |
| Master | C: |
| Secondary | D: |
| ISA Interface Only |  |
| Master | C: |
| Secondary | D: |
| ISA & PCI IDE Interfaces Used Two Drives Used |  |
| PCI Master | C: |
| ISA Master | D: |
| ISA & PCI IDE Interfaces Used Three Drives Used |  |
| PCI Master | C: |
| PCI Secondary | D: |
| ISA Master | E: |
| PCI Master | C: |
| ISA Master | D: |
| ISA Secondary | E: |
| ISA & PCI IDE Interfaces Used Four Drives Used |  |
| PCI Master | C: |
| PCI Secondary | D: |
| ISA Master | E: |
| ISA Secondary | F: |

When drives have multiple partitions on them, partitions beyond the primary partition will be assigned in ascending order, based on available drive letters and other installed drives.  Partitions are assigned to primary partitions first (across multiple drives), and then to secondary partitions.  This means that when there are multiple drives with multiple partitions, the logical drive numbering scheme may result in discontiguous logical partitions on the same physical drive.

For instance, if there is a “Drive 1” with two partitions which is a Master on the PCI IDE interface, a “Drive 2” with three partitions which is a Slave on the PCI interface, and a “Drive 3” with two partitions which is a master on the ISA interface the drive letters will be assigned as follows:

Drive 1

Primary Partition		C:

Secondary Partition		F:

Drive 2

Primary Partition		D:

Secondary Partition		G:

Tertiary Partition		H:

Drive 3

Primary Partition		E:

Secondary Partition		I:

### Hard Disk Sub-Menu

#### Hard Disk Type

Auto Configured -- default

User Definable

Number of Cylinders

Number of Heads

Number of Sectors

If	Hard Disk Type = Auto Configured

Then parameters reported by drive

If	Hard Disk Type = User Definable

Then parameters entered by user

The Number of Cylinders, Number of Heads, and Number of Sectors  fields display what is reported by the drive if Hard Disk Type is set to Auto Configured.  If Hard Disk Type is set to User Definable then these fields are entered by the user. Hard Disk Type should be set to Auto Configured unless the drive was formatted on another system with parameters different than those reported by the drive.

#### Maximum Capacity

Value calculated from Cylinder, Head, and Sector info above

#### Initialization Timeout

Disabled	-default for drives D:,E:,F:

5 Seconds

10 Seconds 	-- default for drive C:

31 Seconds

Initialization Timeout selects the number of seconds the BIOS will wait to determine if an IDE drive is present, after initiating a transaction with the drive.  If a drive is present and responds to the BIOS before the Initialization Timeout period, then the BIOS will proceed, i.e. there is no boot-time penalty for a large value when a drive is present.  If a drive is not present, then the BIOS will wait the Initialization Timeout period before proceeding.  If a drive is present, but does not respond to the BIOS within the Initialization Timeout range, then the BIOS will not see the drive and it will not be accessible.  This value should be set to Disabled for any IDE drives not installed.  This parameter works in conjunction with the Hard Disk Pre-Delay parameter contained in the Boot Options sub-menu of the Main menu.

#### IDE Translation Mode

Standard CHS

Logical Block

Extended CHS

Auto Detected-- default

The IDE Translation Mode parameter controls the way in which the BIOS interacts with the drive in terms of drive geometry.  Proper choice is dependent upon the drive’s size, capabilities, and the operating system (OS) used.

Standard CHS is the translation mode that has been in use for years.  Its use limits IDE capacity to a maximum of 528 Megabytes regardless of the size of the drive used.

Logical Block mode overcomes the 528 MB maximum size limitation imposed by Standard CHS mode. It should be used only when the drive supports LBA (Logical Block Addressing), and the OS supports LBA, or uses the BIOS to access the disk.

Extended CHS mode also overcomes the 528 MB maximum size limitation imposed by Standard CHS mode.   It can be used with drives which are larger than 528 MB, but do not support LBA mode.

Auto Detected allows the BIOS to examine the drive and determine the optimal mode. The first choice is to utilize Logical Block  mode if it is supported by the drive. The second choice is to utilize Extended CHS   mode if the drive topology allows.  If neither of the above two methods are possible, then Standard CHS mode is used.   Important NOTE: When set to Auto Detected the BIOS will detect what the drive is capable of, not the translation mechanism which was used to format the drive.  If a drive is run in a mode other than the mode in which  it was partitioned and formatted, unpredictable results may occur, including data loss.

Different operating systems have different abilities regarding IDE Translation Modes:

UNIX operating systems (as currently implemented) do not support either LBA or ECHS and must utilize the Standard CHS method. UNIX can support drives larger than 528 MB, but does so in its own way.

OS/2 2.1 can support LBA, ECHS or Standard CHS methods . Note that LBA support may require a switch setting on an OS/2 driver in order to operate in that mode. (*)

OS/2 2.0 & Novell Netware can support either ECHS or Standard CHS methods, but not LBA.

DOS & Windows can support LBA, ECHS or Standard CHS methods. If the "Fast 32-bit IDE" option of Windows is utilized, make sure to use the  Standard CHS setting only, unless a version of WDCDRV.386 is being used which supports these advanced geometrys.  You will get better performance NOT using “Fast 32-bit IDE” if using a drive which employs Type F DMA on the ISA interface or Mode 3 PIO on the PCI interface, unless using a version of WDCDRV.386 which supports these advanced modes.

#### Multiple Sector Setting

Disabled

4 Sectors/Block

8 Sectors/Block

Auto Detected -- default

The Multiple Sector Setting  field controls the number of sectors which are transferred by an IDE drive per interrupt generated.  If set to Disabled, then an interrupt will be generated for each sector transferred, even if the OS issues a multi-sector transfer.  If set to 4 Sectors/Block or 8 Sectors/Block then 4 or 8 sectors will be transferred per interrupt (for a request that is as large as, or larger than 4 or 8 sectors).  If Auto Detected is chosen, then x sectors will be transferred per interrupt, where x is the maximum value supported by the drive.  If a drive does not support multiple sectors/block then the BIOS will ignore this setting.  If the drive does not support the specific number (4 or 8) of sectors chosen, then the BIOS will default to the largest number of sectors smaller than the chosen value supported by the drive.

#### Fast Programmed I/O Modes

Disabled

Auto Detected -- default

The Fast Programmed I/O Modes field controls the speed in which Programmed I/O (PIO) transfers occur on the PCI IDE interface.  If set to Disabled, transfers occur at an un-optimized (Mode 0) speed.  If set to Auto Detected, transfers occur at the rate which the drive reports as its maximum (up to and including the maximum mode 3 timing).  This field should only be set to Disabled if a drive is being used which incorrectly reports an ability to function with advanced timings.

### Main Menu Continued

#### Language

English (US)

..

..

The Language field controls the language of the text strings used in Setup and the BIOS.  Any installed language will appear as an option in its own language (for example, German will appear as Deutch).  Only installed languages will appear as options.

#### Boot Options

When the cursor is placed the Boot Options field and it is selected by hitting the Enter Key, the Boot Options Sub-Menu appears allowing parameters which affect the boot process to be set.

### Boot Options Sub-Menu

#### Boot Sequence

C: First, Then A:

A: First, Then C: -- default

C: Only

A: Only

The field controls the order in which the BIOS attempts to boot from the hard and floppy drives.

#### System Cache

Disabled

Enabled -- default

The System Cache field controls both the primary and secondary caches. Setting System Cache Disabled to will hurt performance, but might be required when running software which utilizes software timing loops and needs to be slowed down to execute properly.

#### Boot Speed

Turbo -- default

De-turbo

The Boot Speed field controls the system CPU cycles.  De-turbo mode slows the CPU by disabling system cache and adding increased refresh cycles.  It does not reduce CPU speed to 4.77Mhz or 8Mhz AT class machines.

#### Num Lock

Off -- default

On

The Num Lock field controls whether the keyboard Num Lock function is turned on or off when the system is booted.

#### Setup Prompt

Disabled

Enabled -- default

The Setup Prompt field controls whether the “Press <F1> Key To Enter Setup” message is displayed during POST (Power On Self Test).It does not control access, just the message.

#### Hard Disk Pre-Delay

Disabled

1 Second

2 Seconds

3 Seconds -- default

4 Seconds

5 Seconds

6 Seconds

7 Seconds

The Hard Disk Pre-Delay provides a mechanism which delays the BIOS’s attempt to initialize the first  IDE drive in the system.  It is necessary because certain drives take longer than others to “spin up” and might hang if they are accessed too soon after being powered up or reset.  The delay is only honored for the first drive accessed. This parameter works in conjunction with the Initialization Timeout parameter contained in the Hard Disk Sub-menu of the Main menu.  The two are added together.

It should be noted that this new addition to the BIOS Setup is the result of our speeding up the POST process.

#### Typematic Rate Programming

Default -- default

Override

The parameter controls whether default or user configurable values are used for Typematic Rate Delay and Typematic Rate (see below).  If Default is chosen then Typematic Rate Delay is set to 500 msec and Typematic Rate is set to 15.  If Override is chosen the following two screens appear allowing the user to customize these values.

#### Typematic Rate Delay

250 msec -- default

500 msec

750 msec

1000 msec

The Typematic Rate Delay field controls how long it takes for the keyboard auto-repeat function to start when a keyboard key is held down.

#### Typematic Rate

6 char/sec -- default

8 char/sec

10 char/sec

12 char/sec

15 char/sec

20 char/sec

24 char/sec

30 char/sec

The Typematic Rate Delay field controls the speed characters repeat when you hold down a keyboard key.  The higher the number, the faster the repeat.

### Main Menu Continued

#### Video Mode

The Video Mode  field is informative only.

#### Mouse

The Mouse  field is informative only.  If the system supports a PS/2 style mouse, and the mouse is installed, this field will be displayed.  If the system does not support a PS/2 style mouse, or if it does and the mouse is not installed, this field will not be displayed.

#### Base Memory

This Base Memory field is informative only.   There is another Base Memory field which can be changed in the Advanced Chipset Configuration Sub-Menu of the Advanced menu.

#### Extended Memory

The Extended Memory field is informative only. Amount of system memory above 1 Megabyte

## Advanced Menu

#### Processor Type

#### Processor Speed

#### Cache Size

The Processor Type, Processor Speed, and Cache Size fields are informative only and can not be changed by the user. Cache Size refers to the L2 cache only.

#### Peripheral Configuration

#### Advanced Chipset Configuration

#### Power Management Configuration

#### Plug and Play Configuration

The Peripheral Configuration, Advanced Chipset Configuration, Power Management, and Plug and Play Configuration fields are used to enter sub-menus where these areas are configured.  Press Enter to access.

### Peripheral Configuration Sub-Menu

#### Configuration Mode

Auto -- default

Manual

The Configuration Mode fields controls whether  on board resources are manually or automatically set up.  When set to Auto, on board  system peripherals (IDE drives, Floppy Drive, Serial Ports, Parallel Port) are automatically configured during power up.  When set to Manual, system peripherals must be explicitly configured.  The following 6 fields (PCI IDE Interface to Parallel Port Address) are only user accessible when Configuration Mode is set to Manual.

#### PCI IDE Interface

Enabled -- default

Disabled

#### Standard ISA IDE Interface

Enabled -- default

Disabled

#### Floppy Interface

Enabled -- default

Disabled

The PCI IDE Interface, Standard IDE Interface, and Floppy Interface  fields allow these drive interfaces to be individually enabled or disabled.

#### Serial Port 1 Address

Disabled

COM1, 3F8h -- default

COM2, 2F8h

COM3, 3E8h

COM4, 2E8h

#### Serial Port 2 Address

Disabled

COM1, 3F8h

COM2, 2F8h -- default

COM3, 3E8h

COM4, 2E8h

The Serial Port 1 Address, Serial Port 2 Address fields allow the user to select the I/O ports used by these interfaces.  If a COM port is chosen for one of the Serial Port Addresses, it will not appear in the option list for the other Serial Port Address, i.e. if one of the Serial Port Addresses is set to Disabled, then there will be four choices for the other serial port address.  Otherwise there will be three choices.

#### Parallel Port Address

Disabled

LPT3, 3BCh

LPT1, 378h -- default

LPT2, 278h

The Parallel Port Address fields allow the user to select the I/O ports used by these interfaces.

#### Parallel Port Mode

Compatible -- default

Extended

When set to Compatible, the parallel port will be set up in a mode compatible with the original AT.  When set to Extended the parallel port will be set up in the most advanced mode allowed by the chipset (ex. ECC, ECP).

#### Serial Port 1 IRQ

Interrupt Used

#### Serial Port 2 IRQ

Interrupt Used

#### Parallel Port IRQ

Interrupt Used

The Serial Port 1 IRQ, , Serial Port 2 IRQ and Parallel Port IRQ  fields display the interrupts used by the Serial and Parallel Ports.  These values may not be changed by the user.  If an interface is disabled via the Serial Port 1 Address, Serial Port 2 Address, or Parallel Port Address fields, then the associated interrupt is not displayed, and is available for use by add in cards.

### Advanced Chipset Configuration Sub-Menu

#### Base Memory Size

512KB

640 KB -- default

The Base Memory Size field controls the mapping of addresses between 512K and 640K (80000h-9FFFFh).  If 512KB is chosen then accesses between 512K and 640K are directed to the ISA bus.  When 640 KB is selected accesses between 512K and 640K are directed to system DRAM.  Unless using an ISA card which requires access to this particular address range this field should be set to 640 KB.

#### ISA Bus Speed

Compatible -- default

Enhanced

The ISA Bus Speed field controls the speed of the ISA Bus clock.  If set to Compatible the speed will fall within the limits defined by the IBM AT Technical Reference (6-8.33 Mhz).  If set to Enhanced the speed will be greater than the maximum defined by the IBM AT Technical Reference manual.  Modern ISA cards can operate with the enhanced speeds,  however some older cards can experience difficulties.  ISA performance will be enhanced when ISA Bus Speed is set to enhanced.   The actual value of the Bus Clock when set to Compatible or Enhanced is dependent upon the external frequency of the CPU:

External Frequency	Compatible	Enhanced

60 Mhz			7.5 Mhz	10 Mhz

66 Mhz			8.25 Mhz	11 Mhz

#### Byte Merging

Enabled -- default

Disabled

Byte Merging is a feature which increases performance for older applications which write to video memory in bytes instead of words.  However, this feature is not supported on all PCI video cards.  For best performance this field should be enabled unless a PCI graphics card which does not support this feature is used.  If this field is enabled and used with a card which lacks support video corruption will be seen in some video modes.

#### PCI IDE Prefetch Buffers

Enabled -- default

Disabled

The field is used to disable a set of prefetch buffers in the PCI IDE controller.  Disabling the buffers may be required when using Operating Systems (such as NT) which do not use the BIOS to access the hard and do not disabled interrupts when completing a programmed I/O operation.  Setting PCI IDE Prefetch Buffers to Disabled has negative performance implications.  It is anticipated that Microsoft will provide a patch which will allow the buffers to remain enabled when running NT.

#### ISA LFB Size

Disabled -- default

1 MB

3 MB

7 MB

The ISA Video Linear Frame Buffer Size provides a mechanism for creating a hole in the system memory map.  Accesses made to addresses within this hole will be directed to the ISA bus instead of main memory.  This should be set to Disabled  unless you are using an ISA card which has memory greater than 64K which needs to be accessed by the CPU,  and you are not using the Plug and Play run-time utilities (Configuration Manager & ISA Configuration Utility).

#### ISA LFB Base Address

Dependent upon ISA LFB Size

If	ISA LFB Size	Then	ISA  LFB Base Address

Disabled		N.A.

1 MB`			15 MB

3 MB			13 MB

7 MB			9   MB

The ISA Video LFB Base Address field displays the starting address of the ISA memory hole if a value other than Disabled is chosen for the ISA Video LFB Size parameter.  If  ISA Video LFB Size is set to disabled, then this field does not appear.  This field is informative only, i.e. it cannot be changed by the user.

#### Video Palette Snoop

Disabled

Enabled-- default

The Video Palette Snoop field controls the ability of a PCI graphics cards to “snoop” write cycles to an ISA video card’s color palette registers.   This should normally be set to Enabled.  It should only be set to Disabled  if:

An ISA card connects to a PCI graphics card via the VESA connector

AND	That ISA card connects to a color monitor

AND	That ISA card uses the RAMDAC on the PCI card

AND	The Palette Snooping feature (also called RAMDAC shadowing) is broken on the PCI card.

#### Latency Timer (PCI Clocks)

Number between 0 and 256

66 -- default

The Latency Timer (PCI Clocks) field controls the length of time an agent on the PCI bus can hold the bus when another agent has requested the bus.  Its default value is 66.  In general this should not be changed.  If there are latency sensitive cards in the system (such as audio cards and/or network cards with small buffers) this can be set to a lower value to reduce latency.  If there are no latency sensitive cards in the system, but I/O intensive applications are being run, this can be increased to slightly improve performance.

### Power Management Configuration Sub-Menu

#### Advanced Power Management

Enabled

Disabled -- default

The Advanced Power Management  field enables or disables Advanced Power Management (APM) support in the BIOS.  If set to enabled the BIOS will work with APM aware Operating Systems to manage the power consumed by the CPU,  IDE drives, and graphics cards.  If set to Disabled then none of the other fields in the Power Management Configuration Sub-Menu are displayed.

#### IDE Drive Power Down

Enabled

Disabled -- default

The IDE Drive Power Down field controls whether or not a spin down command will be issued to the IDE drives when the system goes into low power mode.

#### VESA Video Power Down

Disabled -- default

Standby

Suspend

Sleep

The VESA Video Power Down field controls the VESA power management command issued to the graphics card when the system goes into low power mode.  The proper setting is dependent upon the VESA mode(s) supported by your  graphics card.

#### Inactivity Timer (Minutes)

0 to 255

0 -- default

The Inactivity Timer (Minutes) field controls the number  of  minutes that the system will detect no user activity before going into low power mode.

#### Hot Key

User  definable

The Hot Key field is used to define a key which when entered will cause the APM BIOS to ask an APM (Advanced Power Management) aware Operating Systems to go into immediate power managed mode.  There will be some delay, typically about a minute before the system actually goes into power managed mode,  however the speaker will issue 2 descending tones.

If there is a User Password in effect then upon going into power managed mode the Num Lock, Caps Lock, and Scroll Lock LEDs on the keyboard will flash on and off in sequence, indicating to the user that the system is in Secure Mode.   In this case the correct password will have to be entered before the user regains control of the system.  This password will not be echoed to the screen, whether the screen is active or inactive at the time the password is entered (see Set User Password  field in Security Menu).   The following table describes how the Power Down hot key interacts with the Security Hot Key:

|  | Power-Down Hot Key Entered | Security Hot Key Entered |
| --- | --- | --- |
| Power-Down Hot Key Only Enabled | - System powers down @1 minute (can include screen blanking if enabled) - Powers up when any key/mouse activity is detected | N.A. |
| Security Hot Key Only Enabled | N.A. | - System Secure Immediately - LEDs sequence - Input accepted when password typed |
| - Both Hot Keys Enabled | - System Secure Immediately - System powers down @1 minute (can include screen blanking if enabled) - LEDs sequence - Input accepted when password typed - Powers up when any key/mouse activity is detected | - System Secure Immediately - System powers down @1 minute (can include screen blanking if enabled) - LEDs sequence - Input accepted when password typed - Powers up when any key/mouse activity is detected |

### Plug and Play Configuration Sub-Menu

#### Configuration Mode

Use Setup Utility -- default

Use ICU

The Configuration Mode field is used to choose the method in which information about legacy ISA cards (ISA cards which do not have Plug and Play capabilities) is conveyed to the BIOS to allow the BIOS to set up PCI cards and ISA Plug and Play cards without conflicting with the legacy ISA cards.

If Use ICU is chosen, then the BIOS will depend upon information provided by run-time Plug and Play software (Configuration Manager and ISA Configuration Utility) to ensure that there are no conflicts with legacy ISA cards.  Only choose this method if your system is configured with these utilities and they are used. When Use ICU is chosen no other fields are displayed in this Sub-Menu.

If Use Setup Utility is chosen, then the BIOS will depend upon information provided by the user in the fields below to prevent conflicts between Plug and Play cards and legacy ISA cards.  If the Use Setup Utility is chosen then the user should not install and use  run-time Plug and Play software (Configuration Manager and ICU).

#### ISA Shared Memory Size

Disabled -- default

16 KB

32 KB

48 KB

64 KB

The ISA Shared Memory Size field is used to set a block of system memory which will not be shadowed (shadowing is a technique which copies a block of memory from an add in card’s ROM to the same address in  system memory to allow faster access to the code and achieve higher performance).  By default all upper memory is shadowed unless a block is un-shadowed using this and the ISA Shared Memory Base Address fields.

This field should be set to Disabled unless an ISA card is used which communicates with the CPU through a block of shared memory located in the upper memory area.  If set to a value other than Disabled  the ISA Shared Memory Base Address field will appear below.  Note that if a value of  64 KB is chosen, then the ISA Shared Memory Base Address field can only be set to D0000h or below.

#### ISA Shared Memory Base Address

C0000h

C4000h

C8000h

CC000h

D0000h

D4000h

D8000h

DC000h

The ISA Shared Memory Base Address field works in conjunction with the ISA Shared Memory Size.  See above.

IRQ 3*		-default Used By ISA Card

IRQ 4*		-default Used By ISA Card

IRQ 5		-default Used By ISA Card

IRQ 7*		-default Used By ISA Card

IRQ 9		-default Available

IRQ 10		-default Used By ISA Card

IRQ 11 	-default Used By ISA Card

IRQ 15*	-default Used By ISA Card

Available

Used By ISA Card

* These options will not be visible if consumed by on-board resources (IRQ3 Serial Port 1, IRQ4 Serial Port 2, IRQ7 Parallel Port, IRQ 15, both IDE interfaces used).

The IRQ 3 - IRQ 15 fields are used to indicate whether individual interrupts are used by ISA legacy cards.  If not used, then they should be set to Available.  If used they should be set to Used By ISA Card.  Note: All but one can be set to Used By ISA Card.  One must remain available for PCI cards which use interrupts to function.

## Security Menu

There are two types of passwords available, User Password and Administrative Password.  The User password can be used to control access to the Setup Utility, the keyboard upon initial boot, and the keyboard when used in conjunction with the Security Hot Key.  The Administrative Password is used to control access to the Setup Utility.  The following table describes the interaction between these passwords:

| Password In Effect | Enter Setup with User Password | Enter Setup with Administrate Password | Boot |
| --- | --- | --- | --- |
| User Password Only | Can change all fields | N.A. | User Password Required |
| Administrative Password Only | N..A. | Can change all fields | No Password Required |
| Both | Can change limited number of fields | Can change all fields | User or Administrative Password Required |

If the Administrative Password is in effect and Setup is entered with the User Password only the following fields may be changed:

- System Date

- System Time

- User Password

- Security Hot Key

#### User Password is

Disabled -- default

Enabled

#### Administrative Password is

Disabled -- default

Enabled

The User Password is and Administrative Password is fields indicate whether the User and Administrative passwords have been set.  If neither the User Password is or the Administrative Password is fields are set to Enabled neither the Unattended Start or Security Hot Key (CTRL-ALT-) fields will be displayed.

#### Set User Password

When the Set User Password field is chosen the user is presented with one of two sub-menus.  If there is currently no User Password in effect the following menu appears:

Enter New Password_______

Confirm New Password_______

Press Enter or TAB to Accept

Press ESC to Abort

If there is already a User Password in effect, the following menu appears.  To disable the password, use a carriage return as the new password.

Enter Current Password_______

Enter New Password_______

Confirm New Password_______

Press Enter or TAB to Accept

Press ESC to Abort

#### Set Administrative Password

When the Set Administrative Password field is chosen the user is presented with one of two sub-menus.  If there is currently no Administrative Password in effect the following menu appears:

Enter New Password_______

Confirm New Password_______

Press Enter or TAB to Accept

Press ESC to Abort

If there is already an Administrative Password in effect, the following menu appears.  To disable the password, use a carriage return as the new password.

Enter Current Password_______

Enter New Password_______

Confirm New Password_______

Press Enter or TAB to Accept

Press ESC to Abort

#### Unattended Start

Disabled -- default

Enabled

The Unattended Start  field controls the point at which the User Password is required.  If set to Disabled  the User Password is required before the system boots, either from disk or the network.  The user will be prompted for the password with the text string “Enter Password (1):”

If set to Enabled the system will go through automatic boot activity (for instance running CONFIG.SYS and AUTOEXEC.BAT when booting DOS from disk), however the keyboard will be locked and the User Password must be entered to unlock it.  The BIOS does not provide any prompt string so it is suggested that the last thing that the automatic login sequence does is prompt the user for the password.

The Unattended Start field can only be set if a User Password is in effect.

#### Security Hot Key (CTRL-ALT-)

Alpha-Numeric Key

The Security Hot Key (CTRL-ALT-) field can be used to define a key which when entered in conjunction with the CTRL and ALT keys puts the system into secure mode (locks the keyboard).  When this Hot Key combination is entered the Num Lock, Caps Lock, and Scroll Lock LEDs on the keyboard will flash on and off in sequence, indicating to the user that the system is in secure mode (as opposed to being hung).

## Exit Menu

#### Exit Saving Changes

The Exit Saving Changes field is used to exit the Setup Utility saving the current state of all parameters.

When the Enter key is entered the following sub-menu is displayed:

Exit Saving Changes?

Press Enter to Continue

Press ESC to Abort

If the Enter key is selected then the Setup Utility will be exited with all parameters set to their current values.  If the ESC key is selected, then the user is returned to the Exit menu without effecting any change.

#### Exit Discarding Changes

The Exit Discarding Changes field is used to exit the Setup Utility discarding any changes made to Setup parameters.  When the Enter key is entered the following sub-menu is displayed:

Exit Discarding Changes?

Press Enter to Continue

Press ESC to Abort

If the Enter key is selected then the Setup Utility will be exited with changes to Setup parameters lost.  If the ESC key is selected, then the user is returned to the Exit menu without effecting any change.

#### Load Setup Defaults

The Load Setup Defaults field is used to load default values into all Setup parameters, except for the passwords.  When the Enter key is entered the following sub-menu is displayed:

Load Setup Defaults?

Press Enter to Continue

Press ESC to Abort

If the Enter key is selected then defaults are loaded into all Setup parameters, except for the passwords.  If the ESC key is selected, then the user is returned to the Exit menu without effecting any changes.

#### Discard Changes

The Discard Changes field is used undo any changes made to Setup parameters during this Setup session.  When the Enter key is entered the following sub-menu is displayed:

Discard Changes?

Press Enter to Continue

Press ESC to Abort

If the Enter key is selected then any changes made to Setup parameters are discarded.  If the ESC key is selected, then the user is returned to the Exit menu without effecting any changes.