ADSL Ethernet Router

# Installation Guide for Configuring,

### Running & Troubleshooting

ADSL Ethernet Router FLASH FIRMWARE PROGRAM

Version 1.9

- Configuring

Before running DSLFLASH for XXX.exe, please do the following:

XXX refers to the firmware version number.

Eg. 3.2.10 would refer as 3210.

- Go to Network NeighbourHood, right-click and select Properties.

b) 	Select the TCP-IP for the Network Card connected to your modem. Double-click on it. On the IP Address Tab, copy down all the information (if there’s any) on a piece of paper to insert back once the upgrade firmware procedure is done. Then, enter the IP Address and Subnet Mask as shown below: (Example is based on Windows 98 Second Edition)

- Click OK.

- The system would prompt for a restart except Windows 2000 Professional. Click Yes.

- Running

- After restarting, double-click on the DSLFLASH for XXX.exe to activate the flash program.

- Insert the name of the folder you want to unzip all the files to. Click ‘Unzip’ on the next prompt.

- Click ‘OK’ after finish unzipping. (The number of files you see here might differ)

- The DSLFLASH program would run automatically and starts to update your

firmware.

- Please wait for the firmware upgrading to complete.

!!! WARNING !!!!

DO NOT switch off your Modem or PC at this point as it may lead to corruption of firmware and render the modem unusable.

- The first stage waiting time is 35 seconds. After flashing is done, the program would start the second stage flashing.

- Please wait 90 seconds for the second stage to finish.

!!! WARNING !!!!

DO NOT switch off your Modem or PC at this point as it may lead to corruption of firmware and render the modem unusable.

- Finally, the program will check the firmware on your modem and indicate a successful update with the relevant version.

Click 'OK' to quit.

- Congratulations! Your modem is now updated with the latest firmware.

h)	Remember to insert back the IP Address and Subnet Mask (if there’s any), which you have copied earlier to the TCP-IP of your Ethernet Adapter.

3.0	Troubleshooting

Question1: I saw the following window generated after I double-clicked on DSLFLASH for XXX.EXE.

Answer:	Please check through the resolutions proposed on the screen above. If the

problem persists, follow these steps:

- Close the program and run DSLFLASH for XXX.exe again.

- Restart your system and run the program again.

If all fails, contact your ADSL service provider.

Question2: I saw the following status after updating the firmware.

Answer: 	This status shows that your ADSL Ethernet Router is not updated with the correct firmware version. Please try flashing again. Double-click DSLFLASH for XXX.exe to flash again.

If fail again, contact your ADSL service provider.