Driver Installation Guide for SCO UNIX 4.x

================================================

Introduction

============

This document describes the procedures to install the SCO UNIX driver

for AE-200PNP Ethernet Adapter.

SCO UNIX OS does not support Plug&Play feature and	if your computer

doesn't support PnP function also,  you must set the network card in

"Jumperless" mode through our PNPSETUP.EXE program; Otherwise, it can

not work correctly.

The following files will be used in this installation:

Files	      Contents

-----------    -----------------------------------------

driver.o       PnPSCO Driver Program

setup	      Transfer dos format to unix format setup program

info	      Used by netconfig

init	      Used by netconfig

master	      Used by netconfig

node	      Used by netconfig

reconf	      Used by netconfig

remove	      Used by netconfig

space.c	      Used by netconfig

system	      Used by netconfig

Installation Procedures

=======================

Before you start with the installation  process,  make sure that the

SCO UNIX system is properly installed. Similarly,your adapter should

also be properly installed in your machine.

1. Login as root user. (On maintenance mode)

2. (a) If you own installation Diskette in UNIX format :

# custom

Select "Install".

Select "a New Product".

Select "entire" to install all of the packages.

(b) If you own installation Diskette in DOS format :

Put SCO UNIX Driver diskette to floppy A: and use doscp

command to copy the file "setup" for installing SCO UNIX

PnP Driver into UNIX's directories as following :

# cd /

# doscp A:/sco_unix/setup /setup

!! if you don't have the directory ,namely, "a:/sco_unix/4.x"

in the disk, you should modify contents of the setup file

in the floppy. Please check contents of the setup file.

# chmod +x /setup

# ./setup

3. Use netconfig utility

Avaiable options :

# netconfig

The screen will display the following messages, and Select

item (1) to Add a chain  :

1) Add a chain

2) Remove a chain

3) Reconfigure an element in a chain

4) Quit

4. After the above option completed, the screen display next option

messages :

Select item (4) : sco_tcp   SCO TCP/IP for UNIX

5. When the user completed	the  above option, then various adapter

drivers in SCO UNIX occur and select " RTL  PnP  Family Ethernet

Driver board 0" item.

6. Add chain sco_tcp -> pnp0 (y/n)			   TYPE 'y'

7. Enter the internet Address of this interface.

for example : 192.9.100.1

8. Enter the netmask for this interface (default 255.255.255.0).

9. Does the interface use a broadcast address of all 1's (y/n).

default : y

10. Enter the boardcast address for this interface.

default : xxx.xxx.xxx.255			   press ENTER

11. Are these values correct ? (y/n) 		   TYPE 'y'

12. Pseudo ttys are currently configured, do you want to:

1) Add Pseudo ttys

2) Remove Pseudo ttys

Select an option or enter q to quit [q]	   TYPE 'q'

13. Available options:

1) Add a chain

2) Remove a chain

3) Reconfigure a element in a chain

q. Quit

Select option: q

14. Do you want to relink to kernel now ?		   TYPE 'y'

15. Do you want this kernel to boot by default (y/n)    TYPE 'y'

16. Do you want the kernel environment rebuilt (y/n)    TYPE 'y'

17. Reboot unix, type following command.

# shutdown -y -g0